SUBDIVISION

CONTROL

ORDINANCE

for the

VIGO COUNTY AREA

Prepared by the Area Planning Department for Vigo County, Indiana

NOTE:	Original ordinance was adopted by the Vigo County Area Plan Commission on June 12, 1973.  Revised by the Area Plan Commission on March 4, 1981 and July 6, 1988.

TABLE OF CONTENTS FOR SUBDIVISION CONTROL ORDINANCE

FOR THE VIGO COUNTY AREA

Chapter 103 Subdivision Control Ordinance

(Amended December 28, 1988)

6-103-1   Purpose

6-103-2   Authority

6-103-3   Definitions

6-103-4   General Provisions

6-103-5   Compliance

6-103-6   Remedies and Enforcements

6-103-7   Preapplication Conference

6-103-8   Certification of Compliance

6-103-9   Application Review and Hearing

6-103-10  Lot Specifications

6-103-11  Street Specifications

6-103-12  Utilities and Drainage

6-103-13  Specifications for Documents

6-103-14  Soil and Land Use Data

6-103-15  Fees

6-103-16  Release of Bonding

6-103-17  Repealer

6-103-18  Severability Clause

6-103-19  Effective Date

6 - 103 - 1        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 1

Chapter 103

Subdivision Control Ordinance

showing amendments as of (December 28, 1988)

6 - 103 - 1.  Purpose.  The purpose of these regulations is to 		   protect and promote the public Health, Safety, and General Welfare and to provide for:

A.  The proper arrangement of streets or highways in relation

to existing or planned streets or highways;

B.  Adequate and convenient open space for traffic, utilities, 	   	access for fire fighting apparatus, recreation, light, and air;

C.  Guidance of public and private policy and action in order to assure 	adequate and efficient water, sewerage, schools, parks, drainage,

and other public requirements and facilities;

D.  Establishment of reasonable standards of design and minimum  	 	       requirements for the creation, installation, and improvement of physical facilities which are, or will be, maintained for the                                                                                              benefit 	of general public;

E.  Establishment of reasonable standards and procedure for 	 	   	subdivisions resubdivisions, in order to further the 	 	   	orderly layout and use of land; and to insure proper legal 	   	descriptions and monumenting of subdivided land;

F.  Prevention of the pollution of air and water; provision of 	   	drainage facilities and the safeguarding of the water 		   	table; and the encouragement of wise use and management of 	   	natural resources in order to preserve the integrity, 		   	stability, natural beauty and topography, and the value of the land;

G.  Administration of the regulations by defining the powers and duties 	   of approval authorities; and the manner and form of making, filing, and processing of any plat;

H.  The planning of a subdivision is the joint responsibility of the 	    sub divider and the Vigo County Area Plan Commission, the former        having the prime responsibility for the creation of desirable,   	     stable neighborhoods that become an integral part of the entire 	                county.  Subdivision design and utility can enhance or depreciate the character and potentialities of the surrounding areas and stabilize or endanger an individual's investment.

103 – 1

6 - 103 - 1        SUBDIVISION CONTROL ORDINANCE 	    6 - 103 - 3

The commission has the responsibility of helping the subdivider achieve a high standard of excellence in planning of his subdivision and of informing all subdividers of the minimum standards and requirements for subdivision development within Vigo County.

6 - 103 - 2.  Authority.

A.  The Planning Act of 1957 as amended by Indiana Code,

36-7-4-100 through 1200.  (Public Law 309, Acts of 1981) "An Act Authorizing the Establishment of Area Planning Department and Through Their Administration The Development, Through Planning, Of Urban and Rural Areas" enables the city, county and towns to adopt regulations governing plats, replats and subdivision of land within their respective jurisdictions.  The ordinance shall specify the standards by which the commission shall determine whether a plat qualifies for approval.

The ordinance must include standards for:

1.  	Minimum width, depth, and area of lots within the 	 		 		subdivision;

2.  	Street widths, grades, curves and the coordination 					of subdivision streets with existing and planned 						streets and highways;

3.   The adequate provision of water, sewer, and other

municipal services;

4.	The allocation of areas to be used as streets, 						parks, schools, public and semi-public buildings, 					homes, and utilities.

6 - 103 - 3.  DEFINITIONS.

A.	As used in this ordinance, the following terms shall 					have the meanings ascribed to them.

ALLEY.	A strip of land, dedicated to public use, primarily to provide vehicular service access to the side or rear of properties otherwise abutting on a street.

APPLICANT. The owner of land proposed to be subdivided or his   representative.  Consent shall be required from the legal owner of the premises.

BLOCK.	A tract of land bounded by streets, or a combination of streets and public parks, cemeteries, railroad right-of-way, shorelines of waterways, or boundary lines of municipalities.

103 - 2

6 - 103 - 3        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 3

BOND.	Any form of security including a cash deposit, surety bond, or instrument of credit in an amount and form satisfactory to the Plan Commission.  The Plan Commission shall approve all bonds whenever a bond is required by these regulations.

CHANNEL.	A natural or artificial watercourse of perceptible extent, with definite bed and banks to confine and conduct continuously or periodically flowing water.

CITY, COUNTY, TOWN.  The City (county, town) having jurisdiction of the parcel of land under consideration.  The word city shall include town.

COMMISSION.  The Area Plan Commission for Vigo County.

COMPREHENSIVE PLAN OR MASTER PLAN.  A plan for the physical development of the community, prepared and adopted by the Commission, pursuant to State Law, and including any part of such plan separately adopted and any amendment to such plan, or parts thereof.  The document shall show the general location and extent of present and proposed physical development, including, but not limited to housing, industrial and commercial uses, streets, parks, schools, and other community facilities.

COUNTY.  The County of Vigo, Indiana

CONSTRUCTION PLAN.  The maps, drawings, and textual descriptions accompanying a subdivision plat and showing the specific location and design of improvements to be installed in the subdivision in accordance with the requirements enumerated in this ordinance as a condition of the approval of the plat.

COVENANT.	  A private legal restriction on the uses of land contained in the deed to the property and otherwise formally recorded.

CUL-DE-SAC.  "Cul-de-sac stem" means a short minor street, having one end open to motor traffic.  "Cul-de-sac" means the vehicular turn-around at the end of the stem.

CULVERT.	Drain pipes that channels water under a bridge, street, or driveway.

DEDICATION.  The setting apart of land or interests in land for use by the public by ordinance, resolution, or entry in the official minutes as by the recording of a plat.

103 - 3

6 - 103 - 3        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 3

DEFINITIONS GENERALLY.  The following definitions represent the meanings of terms as they are used in these Subdivision Regulations; the singular number includes the plural and the plural, the singular.  The word shall is mandatory.  The word should may be interpreted by the Commission as either mandatory or permissive depending upon the detailed circumstances of each specific application.

DENSITY. A unit of measurement; the number of dwelling units per acre of land.  (1) Gross density - the number dwelling units per acre of the total land to be developed, including public rights-of-way.  (2) Net Density - the number of dwelling units per acre of land when the acreage involved includes only the land devoted to residential uses, excluding public rights-of-way and other public sites.

DEVELOPER.  The owner of land proposed to be subdivided, or his representative.  Consent shall be required from the legal owner of the premises.

DIRECTOR.  Executive Director as defined herein or his designated staff member.

DRIVEWAY.  A private road giving access from a public way to a building on abutting grounds. For the purpose of this ordinance a driveway can be an easement of ingress and egress to a public way when grounds do not abut a public way.

EASEMENT.  An authorization or grant by a property owner to specific person(s) or to the public to use land for specific purposes.

ESCROW.  The arrangement for the handling of instruments or money not to be delivered until specified conditions are met.

EXECUTIVE DIRECTOR.  The executive Director of the Area Planning Department as prescribed by law.

FLOOD OR FLOODWATER.  The water of any lake or watercourse, which is above the banks and/or outside the channel and banks of such watercourse.

FLOOD HAZARD AREA.  Any flood plain district, floodway district, floodway fringe district, AO zone or any combination there as by the Federal Emergency Management Agency in an approved study.

FLOOD PLAIN (FP) DISTRICT.  An area adjoining a river or stream which maybe covered by floodwaters and has been identified as approximate 100-year flood boundaries by the Federal Emergency Management Agency in approved study.

103 - 4

6 - 103 - 3        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 6

FLOODWAY (FW) DISTRICT.  The channel of a river or other watercourse and the adjacent land areas that must be reserved in order to discharge the base flood without cumulatively increasing the water surface elevation more than 1/10 foot and has been identified as approximate 100 year flood boundaries by the Federal Emergency Management Agency in an approved study.

FLOODWAY FRINGE (FF) DISTRICT.  The areas covered by floodwaters which are adjacent to the floodway but are not of sufficient motion or volume to be considered part of the floodway and has been identified as approximate 100 year flood boundaries by the Federal Emergency Management Agency in an approved study.

FRONTAGE.  The length along the street right-of-way line of a single lot, tract, or development area between the side lot lines of the property.  It is that side of a lot abutting a street and ordinarily regarded as the front of the lot.

GRADE.  The slope of a road, street, or other public way, specified in terms of percentage (%).  Example:  One foot of rise in 100 feet would be a one percent (1%) grade.

GOVERNMENTAL ENGINEER.  (1) The County Highway Engineer or his designated representative for the unincorporated area of Vigo County;  (2) The City Civil Engineer or his designated representative for the City of Terre Haute; and (3) A registered professional engineer or his designated representative for the appropriate incorporated town within Vigo County.

IMPROVEMENTS.  Street grading and surfacing, with or without curbs and gutters, sidewalks, crosswalks, water mains, sanitary and storm sewers, culverts, bridges, and street trees.

JURISDICTION.  Jurisdiction of local government means all land within its boundaries and any land outside its boundaries over which it is authorized to exercise powers under State Planning Legislation.

LAND SURVEYOR.  Any person who is licensed in the State of Indiana to practice professional land surveying.

LOT.  A tract, plot, or portion of a subdivision or other parcel of land intended as a unit for the purpose, whether immediate or future, of transfer of ownership for building development.

MARKER.  A long-lived object set in the ground to perpetuate a lot or other secondary subdivision property corner.

MOBILE HOME SUBDIVISION.  A division of land into lots for the purpose of occupant ownership and placement of mobile homes thereon.

103-5

6 - 103 - 3        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 6

MONUMENT.  Any permanent marker either of stone, concrete, cast iron, heavy galvanized iron pipe, (recognizable or readily identifiable) or steel rods, used to identify the boundary lines of any tract, parcel, lot, or street lines.

NON-DEDICATED (ROADS).  Any street or road that is private and not the responsibility of the City or County to perform maintenance on said street or road, and which meet the minimum geometric standards as set forth in this ordinance.

NON-RESIDENTIAL.  A subdivision whose intended use is other than residential, such as commercial or industrial.  Such subdivision shall comply with the applicable provisions of these regulations.

OFF-SITE (OFF PREMISES).  Any premises or structure not located within the area of the property to be subdivided, whether or not in the same ownership of the applicant for subdivision approval.

OPEN SPACE.  A public or private outdoor area expressly set aside for the use and benefit of the public.

OWNER.  Any person, firm association, syndicate, partnership, corporation, or any other legal entity having legal title to the land as recorded with the Vigo County Recorder sought to be subdivided under these regulations.

PERFORMANCE BOND.  An amount of money or other negotiable security paid by the subdivider or his surety to the County Commissioners or City Clerk which guarantees that the subdivider will perform all actions required by the Plan Commission regarding an approved plat, and provides that if the subdivider or his surety will pay damages up to the limit of the bond, or the surety will complete the requirements of the approved plat.

PLANNED UNIT DEVELOPMENT.  A subdivision designed as a combination of residential, commercial, and/or industrial uses planned for a tract of land to be developed as a unit under single ownership or control, which is developed for the purpose of selling individual lots or estates, whether fronting on private or dedicated streets, which may include two or more principal buildings.

PLAT, SECONDARY.  The final and formal presentation of the map, plan for record of a subdivision and any accompanying material, as described in these regulations.

PLAT, PRIMARY.  The preliminary drawing or drawings, described in these regulations, indicating the proposed layout of the subdivision, which is submitted to the Commission for approval.

103 – 6

6 - 103 - 3        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 6

PUBLIC UTILITIES.  All persons, firms, corporations, co-partnerships, municipal authorities providing gas, electricity, water, steam, telephone, sewer, or other services of a similar nature

RESIDENTIAL DEVELOPMENT.  A tract of land that has a house or houses built thereon since the effective date of this ordinance July 16, 1973.  For the purpose of this ordinance the placement of a mobile home constitutes residential development

RESUBDIVISION (REPLAT).  A change in a map for an approved or recorded subdivision plat if such change affects any street layout on such map or an area reserved thereon for public use, or any lot lines; or if it affects any map plat legally recorded prior to the adoption of any regulations controlling subdivisions.

RIGHT-OF-WAY.  A strip of land occupied or intended to be occupied by transportation facilities, public utilities or other special

public uses.  Rights-of-way intended for any use involving

maintenance by a public agency shall be dedicated to the public use by the maker of the plat on which such right-of-way is established.

RIGHT-OF-WAY WIDTH, STREET.  The distance between property lines measured at right angles to the centerline of the street.

ROADWAY.  Way of ingress and egress which serves two (2) or more parties which is greater than 100 feet in length.  An ingress/egress, which is less than 100 feet in length, may, at the discretion of the Commission, be considered a roadway.

SALE OR LEASE.  Any immediate or future transfer of ownership, or any possessor interest in land, including contract of sale, lease, devise, interstate succession, or transfer, of an interest in a subdivision or part thereof, whether by deed, contract, or other written approval.

SECTIONALIZING.  A process whereby an applicant seeks secondary plat approval only on a portion of a plat, which has previously been granted primary approval.

SCREENING.  Any means of protecting an area of land from the adverse visual and audible effects of another area.

SETBACK.  The distance between a building and the nearest street right-of-way line or property line regardless of whether it is the front, side, or rear of the building.  It is an imaginary line established by the zoning ordinance or subdivision ordinance that requires all buildings to be set back a certain distance from property lines.

103 - 7

6 - 103 - 3        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 6

SHOULDER.  That portion of a roadway between the outer edge of the through traffic pavement and the curb or the point of intersection of the slope lines at the outer edge of the roadway and the fill, ditch, or median slope, for the accommodation of stopped vehicles, for emergency use, and for lateral support.

STREETS OF ROADS.  These general terms denoting a public way for purposes of vehicular and pedestrian travel, including the entire area within the right-of-way.  In rural areas, or in urban areas where there is comparatively little access and egress, a way between prominent termini is usually called a highway or road.  A way in an urban area, with or without provisions made for curbs, sidewalks, and paved gutters, is ordinarily called a street.

SUBDIVIDER.  Any person, individual, firm, partnership, association, corporation, estates, trust, or any other group or combination acting as a unit, dividing, or proposing to divide land so as to constitute a subdivision as herein defined, and includes any agent of the subdivider.

103-8

6 - 103 - 3        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 3

SUBDIVISION.

A.  	Division of any tract of land by the owner or his agent into lot(s) for the purpose of commercial, industrial, or residential development.

B.	A re-division of any lot or tract of land by the owner or 				his agent into additional lots for the purpose of 					commercial, industrial, or residential development.  					These regulations shall not apply to the following:

1.	An adjustment of lines as shown on a recorded plat or lot of record which does not reduce the area, frontage, width, depth, or building setback lines of each building site below the minimum zoning requirements of this subdivision ordinance and does not increase the number of lots;

2.	A division of land for agricultural use;

3.	An allocation of land in the settlement of an 						estate of a decedent or a court decree for the 						distribution of property;

4.	The exchange of land for the purpose of 							straightening property boundary lines which does 						not result in the in the creation of a build able lot;

5.	The unwilling sale of land as a result of legal 						condemnations as defined and allowed in the Indiana 					State Law.

SUBDIVISIONS, MAJOR.  All subdivision not classified as minor subdivision, including but not limited to subdivisions of six (6) or more lots, or any size subdivision requiring any new streets or extension of the local governmental facilities, or the creation of any public improvements.

SUBDIVISIONS, MINOR.  Any subdivision containing not more than five (5) lots fronting on an existing street, not including any new street or road, or the extension of municipal facilities, or the creation of any public improvements, and not adversely affecting the remainder of the parcel for adjoining property, and not in conflict with any provision or portion of the Comprehensive Plan, Thoroughfare Plan, Zoning Ordinance, or these regulations.

103-9

6 - 103 - 3        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 4

THOROUGHFARE PLAN.  A plan and maps established by the Governing Body, pursuant to law as a portion of the Comprehensive Plan, showing the location of streets and roads, functionally classified public facilities, utilities and desirable future infrastructure.  The plan is approved, adopted and established by law and any amendments or additions, including those resulting from the filing and approval of subdivision plats, are adopted by the Governing Body as a continuous updating of the Plan.

TOPOGRAPHIC PLAT.  Contour lines delineating elevations superimposed over the subdivision lot and street layout to indicate existing features, structures, drainage ways, etc., and proposed change of drainage.

TRACT.  An area of land having a separate legal description, which said legal description, existed prior to the original effective date of this ordinance.

UTILITIES.  Installations for transmission of water, sewage, gas, electricity, telecommunications and storm water, and similar facilities providing service and used by the public.

VARIANCE.  A modification of the strict terms of the relevant regulations of this ordinance.

6 - 103 - 4  General Provisions.

A.   Official Title.  The regulations shall be known as 					"Subdivision Control Ordinance for the Vigo County Area".

B.	Administration.  The Area Plan Commission for Vigo 					County shall administer these Subdivision Regulations.

C.   Interpretation.  The provisions of these Subdivision 					Regulations shall be held to be the minimum requirements 				adopted for the promotion of public health, safety and 				general welfare of the respective jurisdictional areas.  				The regulations are not intended to repeal, abrogate, 					annul, or in any manner interfere with any existing 					laws, covenants or rules provided.  Should these 						regulations impose greater restriction than is provided 				by existing laws, covenants or rules, the provision of 				these regulations shall prevail.

103-10

6 - 103 - 4        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 4

D.	Jurisdiction.  These Subdivision Regulations shall be 	applicable to all subdivisions hereinafter made of land within the governmental jurisdictions of Vigo County when adopted by ordinance by their respective legislative bodies. Area Plan Commission shall record no plat or replats of a subdivision of land located within the jurisdiction of the Commission until it shall receive an approval.

E.	Modification of Administrative Provisions.  The Area 					Plan Commission may at its discretion modify any and all 				provisions relating to administrative procedures, filing 				procedures, etc., in order to improve its services to the public.

F. 	Fees.  The plan commission shall establish a uniform schedule of fees proportioned to the cost of checking and verifying the proposed plats.  An applicant shall pay the specified fee upon the filing of an application for approval.  (I.C. 36-7-4 Sec. 704)  Section 6-103-15 of this ordinance contains the schedule of fees.  Check or money order shall be made payable to the Area Plan Commission for Vigo County.

------------------------------------------------------------------

Amendment for the unincorporated areas of Vigo County

Amendment for the City of Terre Haute

F.	Fees.  The plan commission shall establish a uniform schedule of fees proportioned to the cost of checking and verifying the proposed plats.  An applicant shall pay the specified fee upon the filing of an application for approval.  (I.C. 36-7-4 Sec. 704)  Section 6-103-15 of this ordinance contains the schedule of fees. Check or money order shall be made payable to the Area Plan Commission for Vigo County.

------------------------------------------------------------------

G.	Effective Date.  This ordinance became effective for 					each jurisdictional governmental area on July 16, 1973.

H.	Severability.  The provisions of this ordinance are 					considered severable, and if any provision is found to 				be unconstitutional it is the intention of the adopting 				authorities that the remainder shall have full force and 				effect.

103-11

6 - 103 - 4        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 4

I.	Proposed Subdivision Plat, Hearing, Notice.  A person 			desiring the approval of a plat shall submit a written 		application for approval in accordance with procedures 		prescribed by the legislative body in the subdivision 			control ordinance.  (I.C. 36-7-4 Sec. 703)  Upon receipt 			of an application for primary approval, the plan 				commission staff shall review the application for 			technical conformity with the standards fixed in the 			subdivision control ordinance.  Within thirty (30) days 		after receipt, the staff shall announce the date for a 		hearing before the plan commission and provide for 			notice in accordance with (I.C. 36-7-4 Sec. 706).  The 		plan commission shall by rule prescribe procedures for 		setting hearing dates and for the conduct of hearings 			(I.C. 36-7-4 Sec. 705).  After the staff has announced a 		date for a hearing before the plan commission or plat 			committee, it shall:

1.  notify the applicant in writing.

2.  give notice of the hearing by publication in 				    accordance with I.C. 5-3-1; and

3.  provide for due notice to interested parties at 			    least ten (10) days before the date set for the hearing.  The plan commission shall, by rule, determine who are interested parties, how notice is to be given to them, and who is required to give that notice (I.C. 36-7-4 Sec. 706)

If after the hearing, the plan commission determines that the application and plat comply with the standards in the subdivision control ordinance, it shall make written findings and a decision granting primary approval to the plat.  An official designated in the subdivision control ordinance must sign the decision.

If after the hearing, the plan commission disapproves the plat, it shall make written findings that set forth its reasons and a decision denying primary approval and shall provide the applicant with a copy.  An official designated in the subdivision control ordinance must sign this decision.

103-12

6 - 103 - 5        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 5

J.	Grandfather Clause.  This ordinance shall not apply to 				any unrecorded subdivision, or land included within such 				subdivision, as to which plat has been prepared by a 					registered land surveyor, or other person duly licensed 				by the State of Indiana to prepare such plats or 						subdivisions, provided that there shall have been a bona 				fide sale	or transfer of a lot, parcel, or tract 					thereof, prior to the effective date hereof, which sale 				or transfer shall have been made or induced after 					reference to such plat, and proved further that the 					owner or owners of the remaining lots, parcels, tracts, 				or land remaining unsold on the effective date hereof 					shall file with the Area Plan Commission of Vigo County, 				and in the Office of the Recorder of Vigo County, 					Indiana, his, her, their, or its affidavit showing such 				facts, within 30 days after the effective date hereof, 				to which affidavit there shall be attached a copy of 					such plat.

K.	Appeals.  Every decision of the Area Plan Commission 					shall be subject to review by certiorari.

L.	Annual Review.  The Area Plan Commission shall review 	the problems encountered during implementation of the ordinance

at least once every year.  The review shall be open to the public and shall be held during a meeting advertised ten (10) days in advance.  The Area Plan Commission to the appropriate legislative bodies for their consideration shall submit any recommendation determined by this review.

6 - 103 - 5  Compliance.

A.	Proceedings Before Approval.  There shall be no sales or 				development prior to approval.  No person, firm, or 					corporation proposing to make a subdivision within the 				jurisdiction of the Commission shall enter into any 					contract for the sale of, or shall offer to sell, any 					lots in said area, or shall proceed with any 						construction work on the proposed subdivision, including 				grading, until the person, firm, or corporation has 					obtained from the Commission written approval of a 					primary plat of the proposed subdivision according to 					the procedures herein outlined.  No transfer of title 					shall be made until secondary approval has been granted 				and plat duly recorded.

B.	Compliance Required.  No subdivisions of land shall be 				permitted unless in accordance with these Subdivision 					Regulations.

103-13

6 - 103 - 5        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 5

C.	No Certificate of Occupancy shall be issued by the 					Building Inspector, or his agent, for any structure on 				any subdivision lots prior to installation and 						completion of all required improvements, including 					grading, as shown on the development plans and approved 				by the Area Plan Commission; except that in the case of 				an asphalt road surface, the installation of the final 				surface coat may be postponed until the end of the 					maintenance period.  The final coat of asphalt shall be 				installed prior to acceptance of road for public 						maintenance.

D.	Application of Regulations.  In planning and developing 				a subdivision, the sub divider or his agent shall comply 				with the general principles and requirements set forth 				in these regulations, and in every case shall pursue 					the procedures outlined in this ordinance with the 					following exceptions:

1.	Subdivision on an Existing Improved Street of Sufficient Width.  In a subdivision of five (5) or less lots wherein lots abut existing improved streets of sufficient width, the Commission may approve or reject the subdivision according to standards set forth herein and may waive the requirements for topographic, street, utility and storm drainage.  Approval of the primary and secondary plats may be concurrent in such instances.

2.	Variances.

a.	Where the Area Plan Commission finds that 							undue hardships or practical difficulties may

result from strict compliance with these 							regulations and/or that the purposes of these 						regulations may be better served by an 							alternative proposal, it may approve variances 						to these subdivision regulations so that 							substantial justice may be done and the public 						interest secured, provided that such variances 						shall not have the effect of nullifying the 						intent and purpose of these regulations; and 						further provided the Area Plan Commission 							shall not approve variances unless based upon 						the evidence presented to it each specific 							case, it is found that:

103-14

6 - 103 - 5        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 6

-  The granting of the variance will not be detrimental

to public safety, health, or welfare, or injurious to 		     other property;

-  The conditions upon which the request for a variance		   is based is peculiar to the property for which the 			   variance is sought and are not applicable generally to 		   other property;

-  Because of the particular physical surroundings, shape, 				or topographical conditions of the specific property 				involved, a particular hardship to the owner will 					result, as distinguished from a mere inconvenience, if 				the strict letter of these regulations is carried out.  				Financial hardship alone does not constitute grounds 				for a variance.

-  The variance will not in any manner vary the provisions 				of the Zoning Ordinance, Comprehensive Plan, or 					Thoroughfare or Major Street Plan which may be in 					effect at the time.

b. In those instances when the requested variance will have

an impact on the design, construction, and maintenance of public facilities, all appropriate public agencies shall be notified and shall be given ample time to comment in writing to the Area Plan Commission.

c.	The Area Plan Commission may require such conditions as will, in its judgment, secure substantially the objectives of the standards or requirements of these regulations.

d. Before approval of a request for a variance is given a                                                                 petition for any variance shall be submitted in writing, by    the Subdivider, at the time that the plat is filed with the Executive Director.  The petition shall state fully the grounds for the application for a variance and all of the facts, which were relied upon by the petitioner.  Each request shall make reference to the specific chapter, section and paragraph of the Ordinance from which a variance is requested.

103-15

6 - 103 - 6        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

6 - 103 - 6  Remedies and Enforcement.

A.	Violations and Penalties.  Any person who violates a 					provision of this ordinance or any regulations herein 					contained, shall be guilty of a misdemeanor and, upon 					conviction, shall be fined not less than ten dollars 					($10.00) and not more than three hundred dollars 						($300.00) for each day's violation.  (Note: these 					penalties are suggested as being reasonable and can be 				varied where conditions seem to warrant).

B.	The Plan Commission, the Board of Zoning Appeals, or any 		designated enforcement official may institute a suit for 		injunction in the circuit court to restrain an 				individual or a unit of government from violating the 			advisory planning law or the area planning law, as the 		case may be, or of an ordinance adopted under the 			advisory planning law or the area planning law, as the 		case may be.

C.	The Plan Commission or Board of Zoning Appeals may also 				institute a suit for a mandatory injunction, directing 				an individual or a unit of government to remove a 					structure erected in violation of the advisory planning 				law of the area planning law, as the case may be, or of 				an ordinance adopted under the advisory planning law or 				the area planning law, as the case may be.

D.	If the Plan Commission or the Board of Zoning Appeals is 				successful in its suit, the respondent shall bear the 					cost of the action.  A change of venue may not be 					granted in such a suit (I.C. 36-7-4 Sec. 1015).

E.   Status of Structures Erected in Violation.

1.	Area Planning.  In an action or proceeding by the 					municipality or county for the taking, 							appropriation, or condemnation of land; or in an 						action against the municipality or county, no 						compensation or damages may be awarded for the 						taking of or injury to any structure erected in 						violation of the advisory planning law or the area 					planning law, as the case may be.

2.	Area Planning.  Furthermore, no compensation or 					damages may be awarded for the taking of or injury

to any structure erected in violation of:

-  An ordinance adopted under the area plan law; or

-	Any prior ordinance superseded by an ordinance 		      adopted under the area planning law (I.C. 36-7-4 Sec. 1017).

103-16

6 - 103 - 6        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

6 - 103 - 7  Preapplication Conference.

A. 	Purpose.  Subdividers are encouraged to seek advice from 				the Area Planning Department on site location, 						development programs and technical assistance on land 					planning, prior to submission, of a primary application.  				The developer should avoid spending time and money in 					making detailed plans or expensive exhibits until the 					Department's advice has been obtained from the 						preapplication conference.  No fee is involved for this 				service.

B.	Handouts to Prospective Subdividers.  The subdivider shall be provided a set of documents, maps and other material, as the Commission deems necessary in order to expedite the processing of his applications.  The documents shall be provided at regular department prices.

C.	Preliminary Sketch Plan.  A Preliminary Sketch Plan should be prepared and submitted to the Department at the Preapplication Conference.  It is not necessary for the sketch to be prepared by a registered land surveyor as the landowner may prepare it.  The sketch must be reasonably accurate as to scale and the dimensions of the parts must add up to the dimensions of the whole.  The sketch may be in pencil or ink on white paper.  The subdivider must be prepared to leave at least one copy with the Department.  The preferred scale is 1" = 50 feet (one inch equals 50 feet).

6 - 103 - 8  Certificate of Compliance.  (Deleted by revisions made and approved the Vigo County Area Plan Commission on 	July 6, 1988.)

6 - 103 - 9  Application Review and Hearing.

A.	Primary Plat Review and Hearing Major Subdivisions.

1.	Major subdivisions are subject to a two (2)-step 						approval process.  Physical improvements to the 						land to be subdivided are authorized by a 							conditional use permit.  Sale of lots (transfer of 					title) is permitted only after Secondary (Final) 						approval is granted.

2.	All primary subdivision applications and plats 						shall be reviewed by the Area Planning Department.

103-17

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

3.   Application.  The subdivider shall file an 							application with the Executive Director for 						approval of the primary plat.  The application shall:

a.   Be made in duplicate and presented at least 				    		     three (3) weeks prior to a regular meeting of  	   		    		     the Commission;

b.	Be made on forms available at the office of 						the Planning Department;

c.	Be accompanied by a fee of Fifty Dollars 							($50.00) plus One Dollar ($1.00) per lot and a 						Ten Dollar ($10.00) legal advertisement fee;

d.	Include all land, which the applicant proposes to subdivide, and all land immediately adjacent to or that directly opposite from the street frontage, with the current use stated on the plat.

e.	Be accompanied by the original, or reproducible copy, and a minimum of seven (7) 	copies of the primary plat as described in these regulations;

f.	Be accompanied by the original, or reproducible copy of the proposed subdivision plat, topographic map and/or any other supporting documents together with a minimum of seven (7) copies of each exhibit.

4.	Primary subdivision applications and plats which do 					not conform to the established subdivision 							standards, but which specifically request variances 					from the published standards, shall be submitted to 					the Area Plan Commission for public hearing and 						consideration.  Said variances to comply with 						Section 6-103-5(D-2) of this Ordinance.

5.	All primary subdivision applications and plats 						shall be submitted to the Area Plan Commission for 					public hearing and consideration.  The applicant or 					his designated representative shall be present at 					the public hearing.

6.   Notice of Public Hearing.  Upon receipt of an application for primary approval, the plan commission staff shall review the application for 	technical conformity with the standards fixed in this subdivision control ordinance.  Within thirty 30) days after receipt, the staff shall announce the date for a hearing before the plan commission and provide for notice.  The plan commission shall, by rule, prescribe procedures for setting hearing dates and for the conduct of hearings. After the staff has announced a date for a hearing before the plan commission it shall:

103-18

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

a.	notify the applicant in writing;

b.	give notice of the hearing by publication in 						accordance with IC 5-3-1; and

c.	provide for due notice to interested parties 						at least ten (10) days before the date set for 						the hearing.  The plan commission shall, by 						rule, determine who are interested parties, 						how notice is to be given to them, and who is 						required to give that notice.

7.	Primary Plat Approval.  After the Area Plan 						Commission has reviewed the primary plat, 							construction plans, reports receive from the 						Subdivision Committee Technical Committee and other 					agencies, and has heard any testimony submitted at 					the public hearing, the applicant shall be advised 					of any required additions.  The Area Plan 							Commission shall by written finding approve, 						conditionally approve, or disapprove the primary

plat.  This decision must be signed by the 							president and attested by the secretary of the 						commission.  The Commission shall also determine 						how secondary approval is to be granted.  The 						Executive	Director shall return one (1) copy of the 					proposed preliminary plat and construction plans to 					the developer with the date of approval, 							conditional approval, or disapproval with the 						reasons therefore in writing, accompanying the plat.

8.	The Area Plan Commission shall take and maintain 						minutes of all hearings.  Said minutes shall 						include, but not be limited to, all pertinent 						comments from commission members, other agencies 						and from the public.  All official dissensions, 						objections and protests shall be recorded.  The 						commission shall maintain, as public record, 						complete minutes of all hearings.

9.	In granting primary approval, the Plan Commission shall determine if the plat or subdivision qualifies for primary approval under the standards prescribed in this ordinance.

10.	As a condition of primary approval of a plat, the 					Plan Commission may specify:

a.	The manner in which public ways shall be laid 						out, graded, and improved;

103 -19

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

b.	A provision for water, sewage, and other

utility services;

c.	A provision for lot size, number, and 								location;

d.	A provision for drainage design; and

e.	A provision for other services as specified in 						this ordinance.

11.	A participating legislative body in this 							subdivision ordinance may reserve the power to 						waive any condition that is imposed upon primary 						approval by the Plan Commission under 6-103-9(A-	10).

Each participating legislative body shall 				prescribe the procedure under which a person may 				apply for a waiver of condition under this 					subsection.

12.	At the discretion of the Executive Director and 						prior to primary hearing before the Plan 							Commission, the applicant may be required to meet 					with a technical committee to review proposed 						construction plans.

The technical committee shall be comprised of:

1. 	Administrative Assistant Board of Health

2.	Highway Engineer, City or County

3.	County Surveyor

4.	Building Inspector, City or County

5.	Soil Conservation Representative

6.	Director Area Planning Department

7.	Applicable Utility Administrators

8.	Fire District Representatives

Recommendations of this technical committee shall be presented to the Plan Commission for their consideration.

13.	Review Process.  The Executive Director shall immediately refer plans to the appropriate agencies of the affected participating jurisdictions for review.  Once these agencies indicate their approval of the construction plans or then (10) working days have elapsed since their distribution. Without a written response, the Executive Director shall stamp the plans approved and return one (1) set to the applicant.  In no event shall secondary 	approval (of the final plat) be given prior to approval of the construction plans.  If additional information or an extension of time is 	required before approval can be given, the agency must give written notice within five (5) working days after the receipt of the plans.  The notice shall state the information requested or reasons for extensions.

103-20

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

14.	Installation of Improvements.  The installation of 					improvements shall be inspected by an appropriate 					participating jurisdiction.  Such inspections are 					required in all instances regardless of whether the 					work is performed before or after secondary 						approval.  Failure to request inspection of work 						performed after the date of this ordinance and 						before secondary approval may be cause for denial 					of secondary approval.

15.	Before actual construction of buildings begins it shall be the responsibility of the Area Planning Department to notify the Building Inspector of any limitations or conditions imposed by the Plan Commission in their granting of approval for a plat.  The notification shall be incorporated into any improvement location permit or conditional permits issued.  It shall be the responsibility of the Building 	Inspector to enforce compliance with those limitations or conditions.

16.	Public Improvements and Performance Bond.

a.	Completion of Improvements.  Subsequent to 							primary plat approval but before the secondary 						plat is signed by the President and Secretary 						of the Commission, all applicants shall be 							required to complete all the streets, curbs, 						sidewalks, sanitary and storm sewers, drainage 						elements, water lines, street signs, and other 						public improvements on the individual lots of 						the subdivision as required in this ordinance, 						specified in the primary subdivision plat, and 						approved by the Area Plan Commission.

b.	The Area Plan Commission, at its discretion, 						may waive the requirement that the applicant 						complete all public improvements prior to the 						approval of the secondary subdivision plat, 						and that, in lieu thereof, the applicant shall 						post bond securable to Vigo County, 								hereinafter referred to as performance bond, 						in an amount equivalent one-hundred and ten

percent (110%) of the estimated cost of 							completion of the required public 								improvements, which shall be sufficient to 							secure the participating jurisdiction the 							satisfactory construction and installation of 						the uncompleted portion of required public 							improvements.

103-21

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

c.	That in lieu of such a bond, the developer may 					submit a certified check made payable to Vigo 					County in an amount equivalent to 110% of the 					estimated cost of completion of the 							uncompleted portion of required public 						improvements.  The Vigo County Auditor shall hold

any such check.

d.	That in lieu of such a bond the subdivider may 						submit a certificate of deposit made out to 						Vigo County and the developer, to be held by 						the County Auditor and in an amount equivalent 						to 100% of the cost of completion of the 							uncompleted portion of required public 							improvements.

e.	That in lieu of such a bond the subdivider may 						submit a certificate that said funds are on 						deposit with a utility company governed by the 						Public Service Commission.

f.	That in lieu of such a bond the subdivider may 						submit documentation that funds are on deposit 						with a state or federally insured bank and 							allocated for specific subdivision 								improvements.  Said amount shall be in an 							amount to 110% of the estimated cost of 							completion of the uncompleted portion of 							required public improvements.

g.	Such performance bond shall comply with all 						statutory requirements and shall be 								satisfactory to the Area Plan Commission 							Attorney as to form, sufficiency, and manner 						of execution as set forth in this ordinance.  						The period within which required public 							improvements must be completed shall be 							specified by the Area Plan Commission in the 						resolution approving the primary subdivision 						plat and shall be incorporated into the bond 						and shall not in any event exceed two (2) 							years from date of secondary approval.  Such 						bond shall be approved by the participating 						jurisdiction as to amount.  The Area Plan 							Commission may, upon proof of difficulty, 							grant an extension of the maximum period of

one (1) additional year, provided that the 							bond submitted for this extension period meets 						all other requirements herein.  The Area Plan 						Commission may at any time during the period 						of such bond accept a substitution of 								principal or sureties on the bond.

103-22

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

17. 	Effective Period of Primary Plat Approval.  Unless 					extended, the approval of a primary plat shall be 					effective for a period of two (2) years at the end 					of which time secondary approval on the subdivision 					must have been obtained and certified by the 						President and Secretary of the Area Plan 							Commission.  Any plats not receiving secondary 						approval within the period of time set forth herein 					shall be null and void and the developer shall be 					required to resubmit a new primary plat for 						approval subject to all new zoning restrictions and 					subdivision regulations.  Upon request of the 						applicant, the Area Plan Commission may extend the 					approval of a primary plat in increments of one (1) 					year beyond an expiration date without further 						notice and public hearing.

18.	Zoning Ordinances.  Every plat shall conform to 						existing zoning ordinances and subdivision 							regulations applicable at the time of approval, 						except that any subdivision which has received at 					the time of primary plat approval shall be exempt 					from any subsequent amendments to the zoning 						ordinance which would otherwise render the plat 						nonconforming as to size, shape, or use.

B.	Primary Plat Review and Hearing Minor Subdivision.

1.	Should the Executive Director, upon examination of 					the application, determine that the proposed 						division is a minor subdivision, and then the 						subdivider shall follow procedures outlined in this 					section.

2.	After a review of a minor subdivision, the 							Executive Director shall determine whether or not 					the application meets the minimum requirements of 					this ordinance.  A meeting shall be scheduled 						within 30 days from receipt of the application with 					the Executive Committee or the Plan Commission for 					a review of approval or disapproval.

3.	The Executive Committee of the Area Plan Commission 					or the Plan Commission may approve or disapprove 						minor subdivisions without public notice and hearing.

4.	The application procedures shall be as outlined in 					6-103-9(A-3b-f) - Primary Plat Review.

5.	The application process for minor subdivisions 						shall be processed as expeditiously as possible.  					However, the Executive Director, Executive 							Committee, or the applicant may at any time refer 					the application to the major subdivision approval 					process.

103-23

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

6.	Not more than a total of five (5) lots may be 						created out of one tract using the minor 							subdivision plat approval process, regardless of 						whether the lots are created at one time or over an 					extended period of time.

7.	If the subdivision is disapproved, the Executive 						Director shall promptly furnish the applicant with 					a written statement of the reasons for disapproval.

8.	The approval of any plat is contingent upon the 						plat being recorded within 90 days after the plat 					is dated and signed by the President and attested 					by the Secretary of the Commission.

9.	Primary and secondary approval may be concurrent.

C.	Secondary Plat Review and Hearing Secondary Approval.

1.	A plat of a subdivision may not be filed with the 					auditor, and the recorder may not record it, unless 					it has been granted secondary approval and signed 					and certified by the president and secretary of the 					Area Plan Commission.  The filing and recording of 					the plat is without legal effect unless approved by 					the Plan Commission.

2. 	The Vigo County Recorder may not record a plat of 					any subdivision within Vigo County unless the plat 					has been approved in accordance with this 							ordinance.

3.	No notice or hearing is required for secondary 						approval.  The Plan Commission may grant secondary 					approval of a plat or may delegate to the Executive 					Committee or the staff the authority to grant such 					secondary approval.

4.	Application Procedures when the Plan Commission requires secondary approval:

a.	Be made in duplicate forms available at the 						office of the Executive Director;

b.	Be made in duplicate and presented at least 						three (3) weeks prior to a regular meeting of 						the Area Plan Commission;

c.	Be accompanied by a fee of Twenty-Five Dollars 						($25.00);

d.	Be accompanied by the original and a minimum 						of seven (7) copies of the final plat which 						shall comply substantially with the primary 						plat as approved.

103 - 24

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

e.	Be accompanied by a minimum of seven (7) 							copies of completed final construction plans 						and/or topographic plat as described in these 						regulations.  Should any modification of these 						plans be made in the actual construction of 						these improvements, "as built" drawings shall 						be submitted upon completion.

5.	Application procedures when the Executive Committees or the Executive Director grants secondary approval:

All of item 4 above shall apply with the exception 					of 4-b.

6.	Endorsements by Other Public Authorities.  The 						secondary subdivision plat shall be accompanied by 					endorsements from appropriate authorities to assure 					the Area Plan Commission that the plat is in 						compliance with all rules, regulations, and 						requirements of local and state authorities, as set 					forth in primary approval.

7.	Final plans shall be submitted in accordance with 					specifications for secondary plat 6-103-13(B).

8.	Approval Procedure:

a.	When approval is required by the Area Plan 							Commission:

Upon receipt of a formal application and all supporting				documents, the Executive Director shall assign						a docket number and the application for secondary						place	 approval on the agenda of the next scheduled					meeting of the Area Plan Commission providing						the submittal of the application is in								compliance with 4-a above.

b.	When approval is delegated to Executive 							Committee:

Upon receipt of a formal application and all supporting 				documents, the Executive Director shall review the 					application to determine if all the requirements of the Area 			Plan Commission's primary approval have been met.  After the 			Executive Director has determined that all requirements have 			been met, he shall set a hearing date	 for the Executive 				Committee of the Area Plan Commission.

103-25

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 9

c.	When approval is delegated to the Executive 						Director:

Upon receipt of a formal application and all supporting documents, the Executive Director shall review the application to 	determine if all the requirements of the Area Plan Commission's primary approval have been met.  After the Executive Director has determined that all requirements have been met, he shall certify this	 to the President and Secretary of the Area Plan Commission, who shall then endorse the plat.

9.	No secondary approval shall be endorsed on the plat 					until a review has indicated that all requirements 					of the Area Plan Commission approval have been met.

10.	Vested Rights.  No vested rights shall accrue to 						any plat by reason of primary or secondary approval 					until the actual signing of the plat by the 						President of the Area Plan Commission, and attest 					action by the Secretary.

a.	All requirements, conditions, or regulations 						adopted by the Area Plan Commission applicable 						to the subdivision, or on all subdivisions 							generally, shall be deemed a condition for any 						subdivision prior to the time of the signing 						of the secondary plat by the President and 							attested to by the Secretary.

b.	Where the Area Plan Commission has required 						the installation of improvements prior to 							signing of the secondary plat, the Area Plan 						Commission shall not unreasonably modify the 						conditions set forth in the secondary 								approval.

11.	Recording of Plat.  The President and secretary 						will sign the original Mylar, drafting paper of the 					subdivision plat, which is to be recorded with the 					County Recorder.  Said signed plat, after 							recording, shall be returned to the Area Planning 					Department which in turn will be released to the 						Surveyor who prepared the plat.

103 - 26

6 - 103 - 9        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

It shall be the responsibility of the subdivider or applicant to file the plat with the County9 Recorder within Ninety (90) days of the date of signature. Simultaneously with the filing of the plat, the 	subdivider shall record in miscellaneous records, any recommendations so deemed necessary by the Area 	Plan Commission.  Otherwise, the plat shall be considered void.  The subdivider shall pay the recording fees.

12.	Sectionalizing of Plats.  Subsequent to granting 						primary approval of a subdivision, the Area Plan 						Commission may permit the plat to be divided into 					two or more sections and may impose such conditions 					upon the filing of the sections as it may deem 						necessary to assure the orderly development of the 					plat.  The Area Plan Commission may require that 						the performance bond or other instrument be in such 					amount as is commensurate with the section or 						sections of the plat to be filed and may defer the 					remaining required performance bond principal 						amount until the remaining sections of the plat are 					offered for secondary plat approval.  The same 						policy shall apply to installation of improvements.

The developer may also file irrevocable offers to 					dedicate streets and public improvements in the 						sections offered to be filed but may defer filing 					offers of dedications for the remaining sections 						until such sections, subject to any conditions 						imposed by the Area Plan Commission, shall be 						granted secondary plat approval.

In the event of approval of sectionalizing, such 						sections as have been approved by the Area Plan 						Commission shall be filed with the County Recorder 					within Ninety (90) days of the date of secondary 						plat signature.

Approval of a plat does not constitute acceptance by any governmental unit having jurisdiction of the offer of dedication of any streets, sidewalks, 	parks, or other public facilities shown on a plat. However, a governmental unit having jurisdiction may accept any such offer of dedication by resolution of the legislative body or by actually exercising control over and maintaining such facilities.

All facilities and improvements with respect to which the				owner makes an offer of dedication to 	public use shall	be

maintained by the owner until the appropriate public authority accepts such offer of dedication.

103 – 27

6 - 103 - 10       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

6 - 103 - 10  Lot Specifications.

A.	General Requirements.

1.	Plats.  In addition to the requirements established 					herein, all subdivision plats shall comply with the 					following rules, laws, and regulations:

a.	All applicable statutory provisions;

b.	Any local zoning ordinances, building and 							housing codes, and all other applicable laws 						of the appropriate jurisdictions in effect;

c.	The official Comprehensive Plan, Thoroughfare 						Plan, and Capital Improvements Program of the 						local government, including all public 							facilities, open space and recreation plans, 						as adopted;

d.	The rules, regulations, and standards of the 						County and State Board of Health, the Natural 						Resources Commission, and other appropriate 						state agencies;

e.	The rules, regulations, and standards of the 						Indiana State Highway Commission if the 							subdivision or any lot contained therein abuts 						a state highway;

f.	All applicable planning and regulatory 							guidelines, including access control or 							driveway manuals, parking and traffic control 						ordinances, and other applicable guides 							legally adopted and published by the local 							governmental units;

g.	The "Indiana Manual on Uniform Traffic Control 						Devices" installation of traffic control 							devices.

B.	Lots.

1.	Special consideration shall be given to lot sizes and shapes where private individual sanitary systems	(Septic Tanks) are to be used.  Particularly acute is the problem of creating lots where both septic systems and water systems are to be developed on the same lot. Because of sever soils; lots larger than prescribed herein may be a necessity.

103 - 28

6 - 103 - 10       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

2.	In all instances the capability to adequately 						accommodate the dwelling unit, water system, 						sanitary system and access shall take precedence 						over minimum lot sizes and minimum lot frontage.  					At no time shall the build able are be reduced below 					2,500 square feet.

3.	Corner residential lots shall be wider than normal 					in order to permit appropriate setbacks from both 					streets.  In general, the lot shape should be 						square or trapezoidal.

C.	One and Two-Family Lot Size Minimums.

1.	Size.  The provisions of the Zoning Ordinance or the following minimums shall control Lot width, depth, and area.

2.	Subdivisions with Conventional Construction.

103 - 29

6 - 103 - 10       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

TABLE A

RESIDENTIAL

LOT SIZE REQUIREMENTS

-------------------------------------------------------------

Type of			Both Sewer &			Neither Sewer

Development		Public Water			Nor Public

Available	and			Water Available

To Be Used

-------------------------------------------------------------

Single-family		 50' A.L.W.*			  100' A.L.W.*

5,500 sq. ft.			 20,000 sq. ft.

Two-family		   	60' A.L.W.*			 110' A.L.W.*

6,600 sq. ft.			 22,000 sq. ft.

_____________________________________________________________

Type of			Sewer Only		        Public Water Only

Development

-------------------------------------------------------------

Single-family		 60' A.L.W.*		       90' A.L.W.*

10,000 sq. ft.			 20,000 sq. ft.

Two-family		      70' A.L.W.*		       100' A.L.W.*

10,000 sq. ft.			 20,000 sq. ft.

-------------------------------------------------------------

* A.L.W. = Average Lot Width / Minimum Frontage

3.	Mobile Home Subdivision (sale of lots) shall be the 					same as single-family.  "Double wide" units shall 					be regarded as prefabricated conventional 							construction for the purpose of this ordinance.

4.	The average lot width should, for most purposes, be 					the minimum frontage also.  However, at no time 						shall the minimum frontage be less than required 						for a driveway with adequate radii for ingress and 					egress.

5.	All lots shall abut on a public or an approved 						private (non-dedicated) street.

6.	Where lots abut a private (non-dedicated) street or 					roadway, the lot lines shall extend to the center 					of the right-of-way; however, the lot measurements 					shall be made from the right-of-way line or the 						outer edge of easement, whichever is greater.

103 - 30

6 - 103 - 10       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

D.	Flood Hazard Areas.

1.	For any proposed subdivision that is located within 					a flood plain, as determined by Flood Hazard Maps 					supplied by Federal Emergency Management Agency, 						the developer or subdivider shall provide the Area 					Plan Commission and the Indiana Department of 	                                          		     	Natural Resources with all the documents required by 					these regulations, and in addition, shall provide 					the Area Plan Commission and Indiana Department of 					Natural Resources with such supporting 							documentation and justifications as may be 							required, to comply with local, state, and federal 					regulations.

2.	No subdivision or any part thereof shall be 						approved if levees, fills, structure or other 						features within the proposed subdivision will 						individually or collectively, significantly 						increase flood flows, heights, or damages.  If only 					part of a proposed subdivision can be safely 						developed, the Area Plan Commission shall limit 						development to that part.

3.	The subdivider or developer shall demonstrate 						conclusively to the Area Plan Commission that the 					proposed development will not present a hazard to 					life, limb, or property; will not have adverse 						effects on the safety, use or stability of public 					ways or drainage channels; and that all approvals, 					permits, and reviews have been received from the 						Indiana Department of Natural Resources when 						required.

E.	Lot Arrangements.

1.	The lot arrangements shall be such that there will 					be no foreseeable difficulties, for reasons of 						topography or other conditions, in securing 						building permits to build on all lots in compliance 					with the zoning ordinance and other regulations and 					providing safe driveway access to buildings on such 					lots from an approved street.

2.	Lot dimensions shall comply with the minimum 						standards of the zoning ordinance, where 							applicable.

103-31

6 - 103 - 10       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

3.	Double frontage and reverse frontage lots shall be 					discouraged except where necessary to provide 						separation of residential development from arterial 					streets or to overcome specific disadvantages of 						topography and orientation.

4.	Lots shall not generally derive access exclusively 					from an arterial or a major collector street.  						Where driveway access from an arterial or major 						collector street may be necessary for several 						adjoining lots, the Area Plan Commission may 						require that such lots be served by a common and 						combined driveway in order to limit possible 						traffic hazards on such street.

5.	Lots shall be laid out so as to provide positive 						drainage away from all buildings and individual lot 					drainage shall be coordinated with the general 						storm drainage pattern for the area.

6.	Topsoil disturbed during subdivision grading 						operations shall be removed from residential lots 					or used as spoil, but shall be redistributed so as 					to provide at least four (4) inches of depth in the 					area between the sidewalks and curbs, and shall be 					stabilized by seeding or planting.

F.	Number and Acreage of Lots by Purpose.  The subdivider 				shall tabulate the total number of lots by Purpose (such 				as single-family, two-family, multi-family, commercial, 				etc.) and specify the net acreage assigned to each 					purpose.

G.	Lot Lines.  Sidelines of lots shall be at approximately 				right angles to straight streets and on radial lines on 				curved streets.  Some variation from this rule is 					permissible, but pointed or very irregular lots should 				be avoided.  The interior angles of the angles of the 					lots should be as orthogonal as practicable.

103-32

6 - 103 - 10       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

H.	Building Line Setbacks.

1.	Building line setbacks shall be as follows unless 					specified in a zoning ordinance:  Interior Lot 						Lines, 5 feet; Street Side Lot Lines, 25 feet; and 					Rear Lot Lines, 11 feet.  The street side building 					line setback shall be measured from the outer line 					of the required five (5) feet wide easement where 					the right-of-way is forty (40) feet in width.  No 					building, foundation, or other structural component 					shall infringe on these setback limits except roof 					overhangs and gable ends with the permitted 						encroachment limited to two (2) feet.  In addition, 					unenclosed stoops or entrances may encroach on the 					street side setback in an amount not to exceed ten 					(10) feet.

2.	These limitations do not apply to Planned Unit 						Development (P.U.D.) where P.U.D. and conventional 					development occur in the same subdivision.

I.	Easements.  Suitable easements shall be provided for the 				installation and maintenance of utilities.  Such 						easements may be located as may be required to properly 				serve the utilities, but shall, whenever possible, be 					along rear and/or side lot lines.  Utility easements 					shall not be less than ten (10) feet in total width,

equally divided between adjacent lots, where such exist,

shall provide reasonable continuity from block to block.

Easements shall be for specific purposes and so stated 				on the plat.

J.	Density.  When density requirements are set forth in the 				zoning ordinance, the subdivider shall calculate the 					following:

1.	The ratio of street right-of-way area to the gross 					(Tract) area;

2.	The ratio of Dwelling Units to the gross (Tract) 						area (Gross Density - D.U./acre);

3.	The ratio of Dwelling Units to the net (Tract area 					minus right-of-way) area (Net Density = D.U./acre 					minus right-of-way).

103-33

6 - 103 - 10       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

K.	Block Arrangements.

1. 	Blocks shall have sufficient width to provide for 					two (2) tiers of lots of appropriate depth.  						Exceptions to this prescribed block width shall be 					permitted in blocks adjacent to major 								transportation facilities, watercourses, and 						industrial, commercial areas, and railroad right-						of-ways.

2.	Whenever practical, blocks along arterials and 						major collector streets shall not be less than one 					thousand (1,000) feet in length.  As a general 						rule, blocks in other residential areas shall not 					be more than one thousand three hundred twenty 						(1,320) not less than four hundred forty (440) feet 					in length.

3.	In long blocks the Area Plan Commission may require 					an easement through the block to accommodate 						utilities, drainage facilities, or pedestrian 						walkways.

4.	Where blocks are developed along arterial streets 					and/or highways that shall contain alleys, those 						alleys shall run parallel to said arterial and not 					perpendicular or radial to it so as to create an 						intersection between the arterial and alley.

L.	Staking Requirements.

1.	All major subdivision boundary corners shall be 						marked with a monument. Monuments shall be firmly 					set, substantial and not subject to settlement, 						frost heave, or other movement.  The monument shall 					be permanent and of such a nature, configuration 						and/or marking as to permit absolute, 								unquestionable identification.

2.	Monuments shall be detectable with a ferrous metal 					finder.  All monuments shall have a dowel or other 					permanent marker and the surface shall have a 						chiseled, incised or embedded identification.  All 					monuments shall be solid, without openings or 						voids.

103-34

6 - 103 - 10       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 10

3.	Monuments of stone or concrete shall be not less 						than four (4) inches in the least dimension and 						shall be nearly square or round.  Encased 							monuments, i.e., concrete in metal pipe or metal 						casing shall not be less than three (3) inches in 					diameter.  Solid metal, cast iron or other 							fabricated monuments shall be not less than one and 					one-half (1 1/2) inches in any least dimension and 					shall expose a surface not smaller than three (3) 					inch diameter or equivalent area at the surface.  					Monuments shall extend to a depth of not less than 					thirty (30) inches below final ground surface 						level.

4.	Intermediate corners, lot corners, reference and/or 					radius points shall be solid iron rods not less 						than five-eights (5/8) inch in diameter, iron pipe 					not less than three-quarter (3/4) inch in diameter 					or other accepted long-lived identifiable object 						which is firmly set, free from movement and which 					can be located with a ferrous metal detector.  						Markers shall extend not less than thirty (30) 						inches into solid ground.

5.	All monuments and markers of a subdivision shall be 					of a similar type material.

6.	The location of all markers shall be determined and 					set by the land surveyor as his fieldwork 							progresses.  The location and type of material 						shall be shown on the secondary plat of the 						subdivision before recording.  Monument shall be 						described for accurate identification.

7.	The quality of draftsmanship for all subdivision 						plats should bear a reasonably professional 						appearance and reveal clear and correct information 					necessary for use in re-establishing boundary 						lines, corners, and other pertinent information to 					the original locations.  All subdivision plats 						shall be drawn to a scale which when reduced to 						half size shall discern clearly this information 						contained thereon.

103 - 35

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

6 - 103 - 11  Street Specifications.

A. 	General Provisions.

1.	In all new subdivisions, streets that are dedicated 					to public use shall be classified as provided in 						Subsection (2).

a.	The classification shall be based upon the 							projected volume of traffic to be carried out 						by the street, stated in terms of the number 						of trips per day;

b.	The number of dwelling units to be served by 						the street may be used as a useful indicator 						of the number of trips but is not conclusive;

c.	Whenever a subdivision street continues an 							existing street that formerly terminated 							outside the subdivision or it is expected that 						a subdivision street will be continued beyond 						the subdivision at some future time, the 							classification of the street will be based 							upon the street in its entirety, both within 						and outside of the subdivision.

2.	The classification of streets shall be as follows:

a.	Minor.  A street whose sole function is to 							provide access to abutting properties.  It 							serves or is designed to serve not more than 						20 dwelling units and is expected to or does 						handle up to 160 trips per day.

b.	Local.  A street whose sole function is to 							provide access to abutting properties.  It 							serves or is designed to serve at least 21 but 						no more than 40 dwelling units and is expected 						to or does handle between 160 and 320 trips 						per day.

c.	Sub collector.  A street whose principal 							function is to provide access to abutting 							properties but is also designed to be used or 						is used to connect minor and local streets 							with collector or arterial streets.  Including 						residences indirectly served through 								connecting streets, it serves or is designed 						to serve at least 41 but not more than 100 							dwelling units and is expected to or does 							handle between 320 and 800 trips per day.

103 - 36

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

d.	Collector.  A street whose main principal 							function is to carry traffic between minor,

local, and sub collector streets and arterial 						streets but that may also provide direct 							access to abutting properties.  It serves or 						is designed to serve, directly or indirectly, 						more than 100 dwelling nits and is designed to 						be used or is used to carry more than 800 							trips per day.

e.	Arterial.  A major street in the city's street 						system that serves as an avenue for the 							circulation of traffic into, out, or around 						the city and carries high volumes of traffic.

f.	Marginal Access Street.  A street that is 							parallel to and adjacent to an arterial street 						and that is designed to provide access to 							abutting properties so that these properties 						are somewhat sheltered from the effects of the 						through traffic on the arterial street and so 						that the flow of traffic on the arterial 							street is not impeded by direct driveway 							access from a large number of abutting 							properties.

3.	Every lot shall have access to it that is 							sufficient to afford reasonable means of ingress 						and egress for emergency vehicles as well as for 						all those likely to need or desire access to the 						property in its intended use.  Access shall be 						limited to well define areas.  Unlimited access 						not permitted.  Anyone wishing to connect a private 					roadway or driveway on to a public street within 						Vigo County shall be required to obtain a Driveway 					Permit.  It is the intent of this ordinance to 						adopt the same driveway permit standards as 						established in the Indiana Department of Highways 					Driveway Permit Handbook.  A Permit Bond is 						established for all commercial drives and new 						roadways into subdivisions.

4.	Whenever a major subdivision that involves the 						creation of one or more new streets borders on or 					contains an existing or proposed Arterial Street, 					no direct driveway access may be provided from the 					lots within this subdivision onto this street.

103 - 37

6  - 103 - 11      SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

a.	All driveway entrances and other openings onto 						streets within the area planning jurisdiction 						shall comply to local or state requirements 						and be constructed so that:

1.	Vehicles can enter and exit from the lot 							in question without posing any substantial danger to					themselves, pedestrians, or vehicles traveling in 					abutting streets and;

2.	Interference with the free and convenient flow of traffic in abutting or surrounding streets is minimized.

3.	The street system of a subdivision shall be coordinated with existing, proposed, and anticipated streets outside the subdivision or outside the portion of a single tract that is being divided into lots (hereinafter, "surrounding streets")as provided in this section.

4.	Collector streets shall intersect with surrounding collector or arterial streets at safe and convenient locations.

5.	Sub collector, local, and minor residential streets shall connect with surrounding streets where necessary to permit the convenient movement of traffic between residential neighborhoods or to facilitate access to neighborhoods by emergency service vehicles or for other sufficient reasons, but connections shall not be permitted where the effect would be to encourage the use of such streets by substantial through traffic.

6.	Whenever connections to anticipated or proposed surrounding streets are required by this section, the street right-of-way shall be extended and the street developed to the property line of the 	subdivided property (or to the edge of the remaining undeveloped portion of a single tract) at the point where the connection to the anticipated or proposed street is expected.  In addition, the permit-issuing authority may require a temporary turnaround to be constructed at the end of such streets pending their extension when such turnarounds appear necessary to facilitate the flow of traffic or accommodate emergency vehicles.  Notwithstanding the other provisions of this subsection, no temporary dead-end street in excess of 1,000 feet may be created unless no other practicable alternative is available.

103-38

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

7.	Streets shall be related appropriately to the topography.  In particular, streets shall be designed to facilitate the drainage and storm water runoff objectives set forth in section 6-103-12-G, and street grades shall conform as closely as practicable to the original topography.

8.   The maximum grade at any point on a street constructed without curb and gutter shall be 6 percent.  On streets with curb and gutter, the grade shall not exceed 6 percent unless no other practicable alternative is available. However, in no case may streets be constructed with grades that, in the professional opinion of the governmental engineer, create a substantial danger to the public safety.  Minimum grade of 0.5 shall be maintained for proper drainage.

9.	Maintenance of right-of-way.  The owner of each lot abutting on Subdivision Street shall maintain the surface area from the edge of his property to the edge of the roadway.  (Edge of the roadway in this instance shall mean the lot side face of curb and gutter or the lot side edge of shoulder.)  The maintenance shall be limited to normal lot or yard maintenance and protective measures to check erosion.  This does not give lot owners authority to alter or block drainage elements.

a.	Wherever there exists a dedicated or platted portion of a street or alley adjacent to the proposed subdivision, the remainder of the street or alley to the prescribed width shall be platted within the proposed subdivision.

b.	Whenever the proposed subdivision contains or is adjacent to a railroad right-of-way or a highway designated as a "Limited Access Highway" by the appropriate highway authorities, provisions shall be made for Marginal Access Road, or a parallel street at a distance acceptable for the appropriate use of the land between the highway or railroad and such streets.

103 – 39

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

12.	Alleys shall be discouraged in residential districts, but should be included in commercial and industrial areas where needed for loading and unloading or access purposes, and where platted, shall be at least twenty-two (22) feet in width.

13.	Intersections of more than two (2) streets at one point shall be avoided.

14.	The following classifications of streets may be constructed with four-foot-wide shoulders and drainage swales on either side in lieu of curb and gutter, so long 	as the street grade does not exceed a 	grade of 6 percent.  Such streets shall be constructed to meet the criteria indicated in the table that follows.

------------------------------------------------------------------

Amendment for the City of Terre Haute

14.	The following classifications of streets may be constructed with four-foot-wide shoulders and drainage swales on either side in lieu of curb and gutter, so long 	as the street grade does not exceed a 	grade of 6 percent.  Such streets shall be constructed to meet the criteria indicated in the table that follows, except that all streets constructed within the corporate limits of the City of Terre Haute, Indiana shall comply with 6-103-11(C-10a).

------------------------------------------------------------------

103 - 40

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

------------------------------------------------------------------

*Minimum Right-of-Way	 	Minimum Pavement

Street Type		 Width (in feet)		 	Width (in feet)

------------------------------------------------------------------

Minor					50			    	    	 18

Local					50			         	 20

Subcollector			55			   		 22

Collector				60			            24

Arterial			   80-120			        12' per lane

10' min. shoulder

------------------------------------------------------------------

*Minimum width of right-of-way may have to be increased to facilitate proper drainage swales or for utilities.

15.	 Except as otherwise provided in Subsection (14) all                streets shall be constructed with curb and gutter and shall conform to 6-103-11(F-7F).  Only standard 90-degree curb may be used, except that roll-type curb shall be permitted along minor and local street within residential subdivisions.

------------------------------------------------------------------

Minimum Right-of-Way	 		Minimum Pavement

Street Type		   Width (in feet)			Width (in feet)

Excluding Curbs

------------------------------------------------------------------

Minor					40			   		 20

Local					40			   		 22

Subcollector			50			   		*24

Collector				60			 		*28

Arterial			    80-120			    	  12' per lane

8' for on street

parking

------------------------------------------------------------------

*Additional pavement width may be required based on speeds exceeding            30 m.p.h.

16.	Whenever a means of pedestrian access is necessary 					from the subdivision to schools, parks, 							playgrounds, or other roads or facilities and that 					such access is not conveniently provided by 						sidewalks adjacent to the streets, the developer 						may be required to reserve an obstructed easement 					of at least 10 feet in width to provide such access.

103 - 41

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE	    6 - 103 - 11

17.	The general layout of streets shall be in the 						following manner:

a.	Subcollector, local, and minor residential 							streets shall be curved wherever practicable 						to the extent necessary to avoid conformity of 						lot appearance.

b.	Cul-de-sacs and loop streets are encouraged so 						that through traffic on residential streets is 						minimized.  Similarly, to the extent 								practicable, driveway access to collector 							streets shall be minimized to facilitate the 						free flow of traffic and avoid traffic 							hazards.

c.	All permanent dead-end streets (as opposed to 						temporary dead-end streets) shall be developed 						as cul-de-sacs in accordance with the 								standards set forth in Subsection (d).  Except 						where no other practicable alternative is 							available, such streets may not extend more 						than 800 feet (measured to the center of the 						turnaround).

d.	The right-of-way of a cul-de-sac shall have a 						radius of 50 feet.  The radius of the paved 						portion of the turnaround (measured to the 							center line of the pavement) shall be 35 feet, 						and the pavement width shall be 12 feet 							without curb and gutter or 18 feet with curb 						and gutter. The unpaved center of the 								turnaround area shall be landscaped and shall 						have a positive drainage outlet under the 							roadway.

e.	Half streets (i.e. streets of less than the 						full required right-of-way and pavement width) 						shall not be permitted except where such 							streets, when combined with a similar street 						(developed previously or simultaneously) on 						property adjacent to the subdivision, creates 						or comprises a street that meets the right-of-						way and pavement requirements of this chapter.

f.	Streets shall be laid out so that residential 						blocks do not exceed 1,000 feet, unless no 							other practicable alternative is available.

103 - 42

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

18.	Streets shall intersect as nearly as possible at 	right angles, and no two streets may intersect at fewer 60 degrees.  Not more than two streets shall intersect at any one point, unless the governmental engineer certifies to the area planning that such an intersection can be constructed with no extraordinary danger to public safety.

19.	Whenever possible, proposed intersections along one 					side of a street shall coincide with existing or 						proposed intersections on the opposite side of such 					street.  In any event, where a centerline offset

(Jog) occurs at an intersection, the distance 						between centerlines of the intersecting streets 						shall be not less than 150 feet.

20.	Except when no other alternative is practicable or 					legally possible, not two streets may intersect 						with any other street on the same side at a 						distance of less than 400 feet measured from 						centerline to centerline of the intersecting 						street.  When the intersected street is an 							arterial, the distance between intersecting streets 					shall be at least 1,000 feet.

21.	Construction and design standards and 	specifications for streets, sidewalks, and curbs and gutters are contained in Street Specification, and all such facilities shall be completed in accordance with these standards.

22.	Except as otherwise provided in this section, all 					lots created after the effective date of this 						section shall about a public street at least to the 					extent necessary to comply with the access 							requirement set forth in Section 6-103-11(A-3).  						For purposes of this subsection, the term "public 					section" includes a preexisting public street as 						well as a street created by the subdivider that 						meets the public street standards of this chapter 					and is dedicated for public use.  Unless the 						recorded plat of a subdivision clearly shows a plat					shall constitute an offer of dedication of such 						right of way.

23.	Architecturally integrated residential subdivisions 					containing 25 or more dwelling units may be 						developed with private roads that do not meet the 					public street standards of this so long as:

103 - 43

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

a.	The proposed development will have direct 							access onto a public street or, if the tract 						has access to a public street only via a 							private road, such private road is improved to 						public street standards, and

b.	No road intended to be private is planned to 						be extended to serve property outside that 							development; and

B.	Provisions for Private Roads

1.	Architecturally integrated subdivisions containing 					any number dwelling units may be developed with 						private roads that do not meet the public street 	               			and sidewalk standards of this chapter but that are 					not intended for dedication to the public so long as:

a.	The proposed development will have direct 							access onto a public street or, if the tract 						has access to a public street only via a 							private road, such private road is improved to 						public street standards,

b.	No road intended to be private is planned or 						expected to be extended to serve property 							outside the development, and

c.	The subdivider demonstrates to the reasonable 						satisfaction of the Plan Commission that the 						private roads will be properly maintained.

2.	A subdivision in which the access requirement of 						Section 6-103-11(A-3) is satisfied by a private 						road that meets neither the public street standards 					may be developed so long as not more than three 						lots have been created out of that same tract.

103 – 44

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

a.	The intent if this subsection is primarily to 						allow the creation of not more than three lots 						developed for single-family residential 							purposes.  Therefore, the Plan Commission may 						not approve any subdivision served by a 							private road authorized by this subsection in 						which one or more of the lots thereby created 						is intended for (i) two-family or multi-family 						residential use or (ii) any other residential 						or nonresidential use that would tend to 							generate more traffic than that customarily 						generated by three single-family residences.

b.	To ensure that the intent of this subsection 						is not subverted, the Plan Commission, among 						other possible options, require that the 							approved plans show the types and locations of 						buildings on each lot or that the lots in a 						residential subdivision served by a private 						road be smaller than the permissible size of 						lots on which two-family or multi-family 							developments could be located or that 								restrictive covenants limiting the use of the 						subdivided property in accordance with this 						section be recorded before final plat 								approval.

3.	Private streets (non-dedicated).  The Area Plan 						Commission, at its discretion, may waive the 						requirements that the applicant complete and dedicate

roads, or waive the requirements for bonding and paving on rural residential subdivisions consisting of 12 lots or less.  Said private roads shall be constructed to meet the geometric standards as set forth in this ordinance except paving requirements.  All private (non-dedicate) roads approved under this subsection shall comply to subsection 4 below.

4.	No final plat that shows lots served by private 						roads may be recorded unless the final plat 						contains the following notations:

a.	"Further subdivision of any lot shown on this 						plat as served by a private road may be 							prohibited by the Vigo County Subdivision 							Control Ordinance."

103 - 45

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

b.	"The policy of the County of Vigo is that, if 						the county or city improves streets that were 						never constructed to the standards required in 						this ordinance for dedicated streets, then 100 						percent of the costs of such improvements 							shall be assessed to abutting landowners."

5.	The recorded plat of any subdivision that includes a 		 private road shall clearly state that such road is a private road.  Further, the seller shall furnish the initial purchaser of a newly created lot served by a private road with a disclosure statement outlining the maintenance responsibilities for the road.

6.	All private roads and access ways shall be designed and constructed to facilitate the safe and convenient movement of motor vehicle and pedestrian traffic.  The provisions of this ordinance, dealing with drainage, Section 6-103-12, shall determine width of roads, use of curb and gutter, and paving specifications.

------------------------------------------------------------------

Amendment for the City of Terre Haute

6.	All private roads and access ways shall be designed and constructed to facilitate the safe and convenient movement of motor vehicle and pedestrian traffic.  The provisions of this ordinance shall determine width of roads, use of curb and gutter, and paving specifications.

------------------------------------------------------------------

7.	Whenever the Plan Commission finds that a means of 					pedestrian access is necessary from the development 					to schools, parks, playgrounds, or other roads or 					facilities and that such access is not conveniently 					provided by sidewalks adjacent to the roads, the 						developer may be required to reserve an 							unobstructed easement of at least 10 feet to 						provide such access.

103 - 46

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

C.	Provisions for Public Roads

1.	Street names shall be assigned by the developer 						subject to the approval of the Planning Department.  					Proposed streets that are obviously in alignment 						with existing streets shall be given the same name.  					Newly created streets shall be given names that 						neither duplicate not are phonetically similar to 					existing streets within the planning jurisdiction, 					regardless of the use of different suffixes (such 					as those set forth in Subsection (2)).

2.	Street names shall include a suffix such as the 						following:

a.	Circle.  A short street that returns to itself.

b.	Court.  A cul-de-sac or dead-end street.

c.	Loop.  A street that begins at the 								intersection with one street and circles back 						to end at another intersection with the same 						street.

d.	Street.

e.	Place.

f.	Avenue.

g.	Drive.

3.	Building numbers shall be assigned by the Area 						Planning Department.

4.	All bridges shall be constructed in accordance with 					the standard and specifications of the Indiana 						Department of Highways, except that bridges on 						roads not intended for public dedication may be 						approved if designed by a licensed architect or 						engineer.

103-47

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE     6  - 103 - 11

------------------------------------------------------------------

Amendment for the unincorporated areas of Vigo County

Amendment for City of Terre Haute

4.	All bridges shall be constructed in accordance with 					the standards and specifications of the Indiana 						Department of Highways.

------------------------------------------------------------------

5.	Utilities installed	in public rights-of-way or 						along private roads shall conform to the 							requirements set forth in Section 6-103-12.

6.	A sight prism shall be provided at all 							intersections.  This prism is an imaginary envelope 					in which no obstruction may be erected which will 					interfere with the vision of approaching drivers.  					The sight prism shall exist at all quadrants of 						each intersection in a subdivision.  The prism is 					measured along the centerline of the approach 						roadways.  The length of each leg of the sight 						prism shall be one hundred ten (110) feet measured 					from the intersection of the two (2) centerlines.  					The prism begins at one and one-half (1 1/2) feet 					above the centerline and extends to a height of 						fifteen (15) feet.

7.	Minimum curb radius at street intersections shall 					be eighteen (18) feet.  Minimum curb radius at 						private driveways shall be ten (10) feet.

8.	Signs.  The subdivider shall provide, at each 						street intersection, substantial, legible, 							permanent street signs.  Signing shall be done in 					accordance with subsection 6-103-11(C-2).  The cost 					of placing signs will be borne by the developer.

9.	Pylons.  At the entrance of all subdivisions, the 					developer or owner shall cause to be erected a 						pylon or sign that will display the official 						subdivision name.

10.	Sidewalks.  Subdivisions which are characterized as being urban, which are contiguous with or extensions to existing subdivisions which have sidewalks and curbs, shall be provided with street curbs and pedestrian walks, and shall be dedicate along with the road.

103 - 48

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE     6  - 103 - 11

------------------------------------------------------------------

Amendment for the City of Terre Haute

a.	The following criteria shall apply to all 							subdivisions within the corporate limits of 						the City of Terre Haute.

(1)	Subdivisions which have 3 lots or more 							per acre shall have sidewalks along both 							sides of the street for pedestrian use.

(2)	Sidewalks shall be constructed within the 							dedicated unpaved portions of the right 							of way of all streets.  Sidewalk 								construction shall comply with 6-103-11(F-7e).

(3)	Concrete curbs are required for all new 							streets.

------------------------------------------------------------------

b.	Whenever curb and gutter construction is used 						with sidewalks on public streets, wheelchair 						ramps for the handicapped shall be provided at 						intersections and other major points of 							pedestrian flow.  Wheelchair ramps and 							depressed curbs shall be constructed in 							accordance with the published standards of the 						appropriate state agency.

11.	Curbs.  Street curbs and walks may also be required 					in those areas where the absence of such 							improvements would create a hazard to the area 						environment and to the health, safety, and welfare 					of its residents.  If said curbs are required, they 					shall be dedicated along with the road.

D.	Thoroughfare Plan.

1.	Wherever the location of a street is indicated in 					the Official Thoroughfare Plan as following an 						existing road or street, or a section or half-						section or other established property line, the 						location of the street lying wholly within a 						subdivision and not designated as following an 						existing road or establishing property line, may be 					varied in its alignment when such variance promotes 					the plan of a neighborhood development unit in 						accordance with good site planning principles, and 					if such alignment provides for the continuity of 						traffic movement.

103 - 49

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

2.	In the absence of any street being designated in 						each section of land, within the jurisdiction, on 					or approximately on the north-south and east-west 					section lines of such section, it is the intent of 					the Official Thoroughfare Plan and this Ordinance 					that Feeder Streets be established on such section 					lines where feasible.

3.	Wherever the location of a street is indicated in 					the Official Thoroughfare Plan as following an 						irregular alignment or a revised alignment or is 						not referenced to an established line, it shall 						follow the alignment shown in the Official 							Thoroughfare Plan.  Such alignment shall be subject 					to a detailed survey, which may be provided by the 					Commission or other public agencies, or by the 						owners of land to be subdivided if required by the 					Commission.  The survey for such street shall be 						subject to the approval of the Commission prior to 					the dedication of the street.

E.	Opening or Widening of Thoroughfare Streets.  Whenever a 				street classified in the Official Thoroughfare Plan is 				to be platted as a part of a subdivision, the required 				right-of-way width for such street shall be as 						specified in the Official Thoroughfare Plan, provided 					that where a street borders a tract of land to be 					subdivided, the owner of such land shall be required to 				plat only one-half of the right-of-way designated for 					such street, measured at ninety (90) degrees to the 					centerline thereof.  In most instances, the subdivider 				will not be expected to develop all the elements of the 				road system contained in the designate standard profile 				(such as the construction of a four-lane divided 						facility) he will, however, be required in all cases to 				provide the necessary right-of-way.

F.	Street Specifications.

1.	General.

a.	All new streets, public or private (non-dedicated), within a subdivision shall be designed to meet the geometric standards set forth elsewhere within this ordinance and the construction of streets shall equal or exceed the minimum requirements set forth herein.

103-50

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

------------------------------------------------------------------

Amendment for the City of Terre Haute

a.	All new streets, public or private (non-							dedicated), within a subdivision shall be 							designed to meet the geometric standards set 						forth elsewhere within this ordinance and the 						construction of streets shall equal or exceed 						the minimum requirements set forth herein, 							except that within the corporate limits of the 						City of Terre Haute all road and drainage 							design shall be in accordance with Indiana 							Department of Highways (IDOH) Road Design 							Manuals I and II.  More specifically, street 						design should be in compliance with current 						AASHTO Interim Guide for Design of Pavement 						Structures.

------------------------------------------------------------------

The standards herein incorporated are 								considered minimum.  Increased construction 						standards may be required for public dedicated 						roads as required by local governmental units.

b.	The developer of a subdivision shall be 							responsible for the design, development, 							construction, and maintenance of all streets 						within an approved subdivision.  This 								responsibility shall include, but not be 							specifically limited to, traffic, pavement, 						walks, curbs, and drainage.

c.	All construction traffic shall, when at all 						possible, be routed to major county roads and 						remain off internal subdivision roads.

d.	Whenever it is determined by the governmental 						engineer that an existing county road has been 						damaged due to heavy construction loads 							attributed to new development, repairs maybe 						required by the developer of the new 								subdivision per recommendations of the 							governmental engineer.

103 - 51

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

e.	The developer's responsibility for maintenance 						of streets shall be continuous until:

-	The street is offered to and accepted by 							the local governmental agency; or

-	The street is offered to and accepted by 							a private agency (such as a homeowners 							association) having adequate resources 							and/or authority to levy funds for the 							maintenance of said streets, or complies 							with Paragraph E-4 of this section.

------------------------------------------------------------------

Amendment for the unincorporated areas of Vigo County

Amendment for the City of Terre Haute

-	The street is offered to and accepted by 							a private agency (such as a homeowners 							association) having adequate resources 							and/or authority to levy funds for the 							maintenance of said streets.

------------------------------------------------------------------

2.	Drawings and Specifications.

a.	The developer shall prepare and submit for 							approval drawings of the proposed street or 						streets within the subdivision as specified in 						this ordinance.

Street drawings shall present the following 						information as a minimum:

-	Proposed street plan and profile showing 							existing ground profile;

-	Typical cross-section;

-	Cross-section;

-	Proposed drainage and drainage relief; 							includes sizes for all pipe culverts 								under roadways and proposed driveways;

-	Soil conditions at the site.

103 - 52

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

b.	The developer shall certify that the streets 						within the subdivision will be built in 							compliance with the requirements set forth in 						this ordinance, or he shall request specific 						construction.

3.	Inspections.

a.	The Area Plan Commission shall provide for 							periodic inspection of required improvements 						during construction to ensure their 								satisfactory completion.

b.	If the local governmental engineer finds upon inspection that any of the required improvements have not been constructed in accordance with the construction standards and 	specifications, the applicant shall be responsible for correcting any errors in construction and completing the improvements in accordance with such standards and specifications.  Wherever a performance bond covers the cost of improvements, the applicant and the bonding company shall be severally and jointly liable for completing the improvement according to specifications.

4.	Testing.

a.	The Area Plan Commission may require physical 						testing of material placed in streets, walks, 						curbs, and other appurtenances during 								construction. 	Such testing, if required, 							shall be done by an agency acceptable to the 						Area Plan Commission.  The costs for testing 						and the submission of test reports shall be 						borne by the developer.

b.	Certification may be used as evidence of 							quality where appropriate.

5.	Street Construction - General.

a.	The standards set forth herein for the 							construction of streets within subdivisions 						are minimum standards.

b.	The minimum construction standards set forth herein shall apply equally to public streets and private		 (non-dedicated) streets, except that surfacing of private (non-dedicated) streets may be waived on private streets that the Plan Commission approved in accordance with 6-103-11(B-3).

103 - 53

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

c.	Construction methods and materials for streets and appurtenances, unless specifically excepted or revised by this ordinance, shall comply with all appropriate provisions of the current edition of the Indiana Department of Highway Standard Specifications, hereinafter referred to as the Standard Specifications.

d.	The Area Plan Commission shall secure and maintain copies of the Standard Specifications, as issued, to be held available to the public for study and reference.

6.	Street Construction - Increased Construction 						Standards.

a.	The Commission may direct additional study and 						design for the construction of streets within 						a subdivision where the recommendations of the 						Soil Conservation Service cannot eliminate 							the unstable ground and drainage conditions 						that are known to exist.

------------------------------------------------------------------

Amendment for the unincorporated areas of Vigo County

Amendment for the City of Terre Haute

a.	The Commission may direct additional study and 						design for the construction of streets within 						a subdivision where the recommendations of the 						Vigo County Soil Conservation Service cannot 						eliminate the unstable ground and drainage 							conditions that are known to exist.

------------------------------------------------------------------

b.	The developer, if so required by the Area Plan 						Commission, shall submit data, plans, and 							specifications for the construction of streets 						and drainage prepared and sealed by a licensed 						professional engineer in the State of Indiana.  						Upon approval by the Area Plan Commission, the 						specific design and specifications, so 							submitted shall become the minimum standards 						for the subdivision.

103 - 54

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 11

7.	Street Construction - Minimum Construction 							Standards.

a.	Sub grade.

-	All vegetation, organic material, trash, 							rubble, and topsoil shall be removed from 							the area to be paved.

-	Pavement construction shall not be placed 							upon soft or yielding sub grades.

-	The City or County Engineer may require 							inspection of the sub grade prior to 								further work.

-	The City or County Engineer may require 							testing and compaction of the sub grade in 							accordance with the Procedures set forth 							in the Standard Specifications, except 							that 100% of maximum dry density per 								AASHTO T99 shall be changed to 95%.

b.	Base Course.

-	Shall the local governmental engineer approve dense-graded, compacted aggregate similar and equal to Standard Specifications size 53, Type 0, or size No. 2 crushed limestone or equivalent as.

-	Compacted thickness of any base course application shall be not less than two (2) inches nor more than four (4) inches.

-	Base course shall be not less than eight 							(8) inches in total compacted thickness 							per standard specifications except that 							100% is changed to 95%.

-	The top four-inch course shall be a 								minimum of number 53-crushed limestone.

-	Base course for use under rigid pavements may be reduced to less than minimum standard thickness provided that evidence of suitability of the sub grade is established and a variance approved by the Area Plan Commission.  Said variance shall be obtained by the appropriate governmental agency and be accepted by them.  Said variance shall be the responsibility by the developer.

103 – 55

6 - 103 - 11       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

c.	Flexible Pavement.

-	Shall be hot asphalt emulsion or hot 								asphalt concrete not less than three (3) 							inches in total compacted thickness,  								(two inch base course and one inch 								surface) placed upon a prepared base 								course.

-	Materials, mix, and application shall be 							in compliance with the appropriate 								requirements of the Standard 									Specifications.

-	Finish and finish tolerances shall be in 							compliance with the Standard 									Specifications.

d.	Rigid (Plain Concrete) Pavement.

-	Shall be not less than six (6) inches in 							thickness placed upon a prepared base 								course.  Five (5) inch minimum for minor 							and local streets in residential 								subdivision.

-	Concrete shall contain not less than 6 							bags of cement per cubic yard, shall have 							a comprehensive strength at 28 days of 							not less than 4000 pounds per square inch 							and shall not have less than 4% nor more 							than 6% air entrained.

-	The preparation, placement, finish, and 							curing of the concrete shall be in 								compliance with the appropriate 									provisions of the Standard Specifications.

-	Longitudinal joints, shall be constructed 							in all pavement more than one lane in 								width, tie bars may be required.

-	Transverse contraction joints shall be 							placed at intervals not to exceed twenty 							(20) feet.  Transverse joints may be 								sawed or formed.  All joints shall be 								sealed as specified by the governmental 							engineer.

103-56

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

-	Load transfer devices may be required at 							contraction joints if specified by the 							governmental engineer.

e.	Walks, Portland cement Concrete.

-	All walks shall be of Portland cement 								concrete as specified for rigid pavement.

-	Walks shall not be of less than four (4) 							inches thick, four (4) feet in width, and 							placed upon a prepared sub grade or base.

-	Preparation, placement, finish, and 								curing of walks shall be in compliance 							with the appropriate provisions of the 							Standard Specifications.

-	Transverse joints shall be sawed, tolled, 							or formed in all walks at intervals of 			    				not more than five (5) feet.

-	All walks shall have a finish surface, which will not become slippery.

f.	Curbs, Portland Cement Concrete.

-	All curbs and gutters shall conform to IDOH							standard specifications or combined curb							and gutter.  Maximum joint spacing shall be							sixteen (16) feet or shall match joints in							concrete pavement when concrete pavement is used.

-	Whenever individual lot access ways are installed, the entire curb and gutter 	section shall be removed to the nearest joint, and a depressed curb shall be installed.

-	No material is to be placed upon the curb 							and gutter to allow lot access.

6 - 103 - 12  Utilities and Drainage.

A.	General Provisions.  Adequate drainage and utilities 					shall be provided to all subdivisions.  It is the intent 				of this ordinance to provide for uniform placement of

utilities and drainage elements within rights-of-way and easements.

103-57

6 - 103 -12        SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

In any case in which a developer installs or causes the 				installation of water, sewer, electrical power, 						telephone, or cable television facilities and intends 					that such facilities shall be owned, operated, or 					maintained by a public utility or any entity other than 				the developer shall transfer to such utility or entity 				the necessary ownership or easement rights to enable the 				utility or entity to operate and maintain such 						facilities.

B.	Sewage Disposal System.  Every principal use and every 				lot within a subdivision shall be served by a sewage 					disposal system that is adequate to accommodate the 					reasonable needs of such use or subdivision lot and that 				complies with all applicable health regulations.

1.	Whenever it is legally possible and practicable in 					terms of topography to connect a lot with a 						sanitary sewer line by running a connecting line 						not more than 300 feet from the lot to such line, 					then no use of sewage disposal service may be made 					of such lot unless connection is made of such line.

2.	Connection to such sewer line is not legally possible if, in order to make connection with such 	line by a connecting line that does not exceed 300 feet in	length, it is necessary to run the connecting line over property not owned by the owner of the property to be served by the connection, and after, diligent effort, the easement necessary to run the connecting line cannot be reasonably be obtained.

3.	For purposes of this section, a lot is "served" by 					sanitary sewer line if connection is required by 						this section.

4.	Primary responsibility for determining whether a 						proposed development will comply with the standard 					set forth in Section B often lies with an agency 						other than the Plan Commission, and the developer 					must comply with the detailed standards and 						specifications of such other agency.  The relevant 					agencies are listed in Subsection 5.  Whenever any 					such agency requires detailed construction or 						design drawings before giving its official to the 					proposed sewage disposal system, the authority 						issuing a permit under this chapter may rely upon a 					preliminary review by such agency of the basic 						design elements of the proposed sewage disposal 						system to determine compliance with Section B.

103 - 58

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

However, construction of such system may not be 						commenced until the detailed plans and 							specifications have been reviewed and any 							appropriate permits issued by such agency.

5.	In the following table, the column on the left 						describes the type of development and the column on 					the right indicates the agency that must certify to 					the Plan Commission whether the proposed sewage 						disposal system complies with the standard set 						forth in Section B.

IF				  	THEN

a. 	Lots within the sub-divisions 		No further certification

are to be served by simple 			is necessary. 				connection to existing public

sanitary sewer lines or lines

of a previously approved private

system:

b.	Lots within the subdivision			The Sanitary District Engi- are to be served by the public 		neer must certify to the sanitary sewer system but the 		Plan Commission that the developer will be responsible for	proposed posed system meets

installing the necessary additions 	the sanitary districts

to the city system.				specifications and will be                              							accepted by the sanitary 								district. (A

"Preconstruction Agreement")                                                                                                                               									must be obtained from the

Board of public works & 										Safety.

c.	Lots within the subdivision 		The County Health Department

are to be served by a sewage		(VCHD) must certify that the

treatment system that has not 		proposed system complies been approved,that has a design       	with all applicable state capacity of 3,000 gallons or		and local health Regulations.  less,and does not discharge into		If each lot within the

surface waters:					subdivision is to be served

by a separate on-site disposal system, the VCHD

must certify that each lot shown on a major subdivision

primary plat can probably be served and each lot on a major or minor subdivision secondary plat can be servedby an off site disposal system.

103 – 59

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 – 12

IF						      THEN

d.	Lots within the sub- 	 			The Department of Environ-

division are to be				mental Management (IDEM) must

served by a privately			certify that the proposed

operated sewage treat-			system complies with all

ment system (not previously			applicable state regulations.

approved)that has a design			( A “Permit to Discharge”

capacity in excess of 			 must be obtained from IDEM).

3,000 gallons or that

discharges effluent into

surface waters:

C.	Water.

Every principal use and every lot within a subdivision 				shall be served by a water supply system that is 						adequate to accommodate that the reasonable needs of 					such use or subdivision lot and that complies with all 				applicable health and fire protection regulations.

1.	The subdivider shall provide the subdivision with a 					complete water main supply system which shall be 						connected to an existing approved municipal or 						community water supply, where such system is 						available within 1,000" radius of the tract 						boundaries.  Where a public water main is 							accessible the subdivider shall install adequate 						water facilities including fire hydrants, subject 					to the specifications of State and Local 							authorities.

2.	An individual water supply may be installed on each 					lot in the subdivision in accordance with the 						minimum requirements of the Indiana State Board of 					Health (refer to Bulletin No. S.E. 7, Safe Water 						Supplies, current issue) if C-1 above does not 						apply.  The responsibility of individual water 						supplies will be that of the developer for each 	lot.

3.	The plans for the installation of a water main supply system shall be provided by the subdivider and approved by the Indiana State Department of Health (refer to Regulations HSE 5, I.S.B.H.).  Upon completion of the water supply installation the plans for such system as built shall be filed with the Area Plan Commission.

103 - 60

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

4.	The location of all water supply improvements 					(including fire hydrants) shall be shown on the 					primary plat, and the cost of providing and 					installing shall be included in the performance bond.

D.	Fire Protection.

1.	Every development that is served by a public water 					system shall include a system of fire hydrants 						sufficient to provide adequate fire protection for 					the buildings located or intended to be located 						within such development.  Design standards shall 						conform to the National Fire Protection Association 					(NFPA) standards, or local requirements.

2.	The presumption established by this ordinance is 						that to satisfy the standard set forth in 							Subsection (1), fire hydrants must be located so 						that all parts of every building within the 						development may be served by a hydrant by laying 						not more than 500 feet of hose connected to such 						hydrant.  However, the fire chief may authorize or 					require a deviation from this standard if in his 						professional opinion another arrangement more 						satisfactorily complies with the standard set forth 					in Subsection (1).

3.	The fire chief shall determine the precise location 					of all fire hydrants, subject to the other 							provisions of this section.  In general, fire 						hydrants shall be placed six feet behind the curb 					line of publicly dedicated streets that have curb 					and gutter.

4.	The fire chief shall determine the design standards 					of all hydrants based on fire flow needs.  Unless 					otherwise specified by the fire chief, all hydrants 					shall have two 2 1/2 inch hose connections and one 					4 1/2 inch hose connection.  The 2 1/2 inch hose 						connections shall be located at least 21 1/2 inches 					from the ground level.  All hydrant threads shall 					be national standard threads or specifications 						approved by the district fire chief.

5.	Water lines that serve hydrants shall be at least 					six-inch lines when possible, and unless no other 					practicable alternative is available, no such lines 					shall be dead-end lines.

103 – 61

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

6.	The location of the hydrants and size of the mains shall be indicated on the topo plat and shall conform to accepted standards and shall be reported to the appropriate township trustee or agency responsible for fire protection.  The developer's responsibility for maintenance of the hydrants shall be continuous until the hydrants are accepted by the local governmental agency.  The local 	governmental agency shall accept maintenance of fire hydrants within a reasonable period of time.

------------------------------------------------------------------

Amendment for the City of Terre Haute

5.	Fire Protection

a.	Every development that is served by a public 						water system shall include a system of fire 						hydrants sufficient to provide adequate fire 						protection for the buildings located or 							intended to be located within such 								development.  Design standards shall conform 						to the National Fire Protection Association 						(NFPA) standards, local requirements.

b.	The presumption established by this ordinance 						is that to satisfy the standard set forth in 						Subsection (a), fire hydrants must be located 						so that all parts of every building within the 						development may be served by a hydrant by 							laying not more than 500 feet of hose 								connected to such hydrant.  However, the fire 						chief may authorize or require a deviation 							from this standard if in his professional 							opinion another arrangement more 								satisfactorily complies with the standard set 						forth in Subsection (a).

c.	The fire chief shall determine the precise 							location of all fire hydrants, subject to the 						other provisions of this section.  In general, 						fire hydrants shall be placed six feet behind 						the curb line of publicly dedicated streets 						that have curb and gutter.

d.	The fire chief shall determine the design 							standards of all hydrants based on fire flow 						needs.  Unless otherwise specified by the fire 						chief, all hydrants shall have two 2 1/2 inch 						hose connections and one 4 1/2 inch hose 							connection.  The 2 1/2 inch hose connections 						shall be located at least 21 1/2 inches from 						the ground level.  All hydrant threads shall 						be national standard threads or specifications 						approved by the district fire chief.

103 - 62

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

e.	Water lines that serve hydrants shall be at 						least six-inch lines, when possible, and 							unless no other practicable alternative is 							available, no such lines shall be dead-end 							lines.

f.	The location of the hydrants and size of the 						mains shall be indicated on the topo plat and 						shall conform to accepted standards and shall 						be reported to the appropriate township 							trustee or agency responsible for fire 							protection.  The developer's responsibility 						for maintenance of the hydrants shall be 							continuous until the hydrants are accepted by 						the local governmental agency.  The local 							governmental agency shall accept maintenance 						of fire hydrants within a reasonable period of 						time.

------------------------------------------------------------------

D.	Gas Lines.  Where public gas is to be used the size and 				location of all transmission and distribution lines 					shall be indicated.  Where the location is not 						explicitly defined, the Commission may designate the 					location of all lines in public right-of-way utility 					easements.

E.	Electrical Systems.

1.	All electrical transmission line locations within 					residential subdivisions shall be approved by the 					Commission.

2.	All electrical distribution lines within the public 					right-of-way shall be approved by the Commission as 					to their locations.

3.	All electrical sub-stations within the subdivision 					or immediately adjacent to the subdivision shall be 					adequately landscaped to provide visual screening.

4.	Where underground distribution and/or service is to 					be provided at the lot fronts, a utility easement 					for either or both the distribution lines and 						transformer pit shall be provided within the front 					yard setback area.

F.	Telephone and Cable TV.

1.	Where applicable, telephone and cable TV lines 						shall be located on the same poles as the 							electrical distribution system.

103 - 63

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

2.	All telephone or cable TV lines placed within the 					public right-of-way shall be approved by the 						Commission as to location in the right-of-way.

G.	Storm Drainage.

1.	The subdivider shall provide the subdivision with 			an adequate storm water sewer system whenever curb 			and gutter is installed and/or whenever the 				evidence available to the Commission indicates that 			the natural surface drainage is inadequate.  When 			the surface drainage is adequate, easements for 				such surface drainage shall be provided.

2.	In a subdivision where curbs and gutter are not 						provided, the subdivider shall furnish one of the 					following types of improvements to facilitate 						roadside drainage and to assure suitable entrances 					for private driveways, which are proposed to 						intersect the roadway:

a.	A 20 feet long corrugated (non-spiraled) pipe 						or plastic pipe placed at all driveways, sized 						according to the calculated amount of storm 						water flow, but not less than 12 inches in 							diameter.  Minimum cover over any culvert will 						be twelve (12) inches, or

b.	A properly dipped or swale concrete pavement, 						fourteen (14) feet in length, eight (8) feet 						in width and six (6) inches thick, designed so 						as not to create a hazard to the under parts of 						automobiles, at the entrance of each driveway.

3.	Open ditches are permitted in special cases if 						approved by the Commission.  They should be used, 					however, only when no other recourse is available.

4.	In any subdivision where surface or subsurface 						drainage runoff must flow over or run through 						adjacent lot(s), the subdivider shall provide 						adequate easements for said drainage way and shall 					petition to the Drainage Board for their acceptance 					of maintenance or the easement and maintenance 						responsibility shall be turned over to a private 						agency (such as a homeowners association) which has 					authority to levy funds to maintain said drainage 					system.

5.	The following shall apply when the responsibility 					of maintenance for drainage areas are that of the 					Drainage Board:

103 - 64

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

a.	In order to ensure the maintenance of properly 						designed and installed drainage systems, the 						following paragraphs shall be required as a 						provision of the restrictive covenants of all 						secondary plats.  Said signed copy of this 							covenant shall be filed with the County 							Surveyor.

1.	Drainage swales (ditches) along dedicated roadways within the right-of-way, or on dedicated drainage easements, are not to be altered, dug out, filled in, tiled, or otherwise changed without the written permission of the Vigo County Drainage Board.  Property owners must maintain these swales as sodded grass ways, or 	other non-eroding surfaces.  Water from roofs or parking areas must be contained on the property long enough so that said such water will not damage drainage swales or ditches.  Driveways may be constructed over these swales or ditches only when appropriate sized culverts are installed.

2.	Any property owner altering, changing, or 							damaging these drainage swales or ditches 							will be held responsible for such action 							and will be given 10 days notice by 								registered mail to repair said damage, 							after which time, if no action is taken, 							the Vigo County Drainage Board will cause 							said repairs to be accomplished, and the 							bill for such repairs will be sent to the 							affected property owner for immediate 								payment.

6.	The Vigo County Drainage Board will accept 							petitions for urban drains.  The County Surveyor 						shall review construction drawings submitted.  The 					design standards will be in accordance with the 						approved standards of the Vigo County Surveyor.  						The Vigo County Drainage Board shall follow 						procedures as established in the Drainage Code of 					Indiana for assessments and maintenance of all 						drainage areas accepted by them.

7.	The Vigo County Drainage Board will not have 						jurisdiction or provide maintenance on individual 					lots for surface swales or subsurface tile drains.

103 - 65

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

8.	Design Standards.  Run-off quantities shall be 						computed for the area of the parcel under 							development plus the area of the watershed flowing

into the parcel under development.  Methods 						acceptable to the Vigo County Surveyor shall be 						used for calculations.

a.	The minor drainage system (such as inlets, 							manholes, street gutters, roadside ditches, 						swales, sewers, and small channels which 							collect storm water run-off must accommodate a 						10-year rainfall intensity without flooding to 						the tops of the curbs provided, however:

1)	Open channels carrying greater than 30 							cubic feet per second shall be capable of 							accommodating a 50-year rainfall 								intensity within the drainage easement, and

2)	Culverts shall be capable of 									accommodating a 25-year rainfall 								intensity when crossing under a road which:

a) carries a higher volume of traffic 						  		 than other adjacent routes, or has the 					  		 potential for carrying such volumes, and

b) is a part of a pattern of arterial 						  		 routes for the entire urban area, and

c) is oriented primarily to moving 						  		 traffic rather than to serving 							  		 abutting land use.

b.	The major drainage system.  Drainage systems 						carrying runoff from an area in excess of one 						square mile shall be designed in accordance 						with Indiana Department of Natural Resources 						Standards.

c.	Where the Vigo County Drainage Board is 							responsible for maintenance of the drainage 						system, access easements of 25 feet from the 						bank on each side of the open channel must be 						dedicated to the Vigo County Drainage Board.

103 - 66

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

9.	The subdivider shall provide a drainage plan.  The 					plan shall show as a minimum:

a.	Size and location of all drainage elements;

b.	Contouring for drainage, with calculations for 						culvert and channel size;

c.	Storm water basins if they are to be provided;

d.	Drainage of all street right-of-way;

e.	Drainage of all public land, dedicated public 						land and utility easements;

f.	Type of storm and runoff calculations.

10.	Where a subdivision is traversed by an existing 						watercourse, drainage way, channel or stream, 						adequate area shall be reserved as an easement for 					maintaining, cleaning, and improving the 							drainage way.  The drainage way easement shall be of 					width sufficient to accommodate the drainage way as 					improved for subdivision use.

11.	The right-of-way for all legal drains shall conform 					to Section 601 of the Indiana Drainage Code.

12.	Subsurface Drainage.  Lots on soils having a severe 					limitation for development due to seasonal high 						table as deter-mined by the National Cooperative 						Soil Survey of Vigo County shall be developed where 					practicable so that each lot can be provided with 					an outlet for perforated subsurface drainage tile.  					When it is impossible for each lot to have an 						outlet for subsurface drain-age tile on their						lot then the developer shall cause to be installed 					a public perforated subsurface drainage system that 					each lot can connect to.  Such public tile system 					shall be required where two or more lots are 						dependent upon the subsurface tile for overcoming 					the seasonal high water table.  Adequate easements 					shall be provided for maintenance.

Where a sealed storm drainage system is installed 					on sever soils, one subsurface outlet into the 						storm sewer shall be provided for each lot to 						connect subsurface drainage tile.

103 - 67

6 - 103 - 12       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 13

------------------------------------------------------------------

Amendment for the City of Terre Haute

13.	In addition to the above stated requirements, the 					following shall apply to all subdivisions within 						the corporate limits of the City of Terre Haute, 						Indiana.

General.  The Plan Commission shall not recommend for approval any plat of a subdivision, which does not make adequate provisions for storm or floodwater runoff.  The drainage system shall be separate and independent of any sanitary sewerage system.  Storm sewers, where required, shall be designed according to the methods recommended by the local City Engineer.  A copy of the design computations shall be submitted along with plans.  	Inlets shall be provided so that surface water is not carried across or around any intersection, nor for a distance of more than 400 feet in any gutter.  Surface water drainage patterns shall be shown for each lot and block.

Requirements for Storm Sewers.  The applicant shall 					be required by the Plan Commission to carry away 						pipe, where practicable, any spring or surface 						water that may exist either previously to, or as a 					result of, the subdivision.  Such drainage 							facilities shall be located in the road right-of-						way where feasible, or in perpetual unobstructed 						easements of appropriate width.

Where a public storm sewer or natural outlet is 						accessible, the applicant shall install storm sewer 					reasonable distance, adequate provision shall be 						made for the disposal of storm waters subject to 						the specifications of the local Governmental 						Engineer.

Where conditions exist that would require a storm 					sewer size larger than what is normally required 						for a particular size subdivision, the local 						government shall bear the additional cost for the 					oversized facility, over and above what would be 						required for the subdivision alone.

103 – 68

6 - 103 - 13       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 12

If the Commission determines that a connection to a 					public storm sewer will eventually be provided as 					shown in existing local plans and programs, the 						developer shall make arrangements for future storm 					water disposal in the subdivision, by a public 						sewerage system, at the time the plat receives 						final approval.  Provision for such connection 						shall be incorporated by inclusion in the 							performance bond required for the subdivision plat.

------------------------------------------------------------------

H. 	Combined Sewers.  The use of a single sewer system for both sanitary and storm effluent is prohibited.

6 - 103 - 13  Specifications for Documents.

A.	Primary Plat.

1.	General.  A licensed land surveyor at a scale of shall prepare the primary plat not more than one (1) inch equals one (100) hundred feet.  It may 	be prepared in pen or pencil and the sheets shall be numbered in sequence if more than one (1) sheet is used.  All sheets shall be of such size as is acceptable for	filing in the office if the County Recorder, but shall	not be larger than twenty-seven inches by thirty-six inches (27" x 36"), The map prepared for the primary may be drawn in drafting paper or reproducible Mylar.

2.	Name of Subdivision.

a.	Name of the existing subdivision if properly 						is within an existing subdivision.

b.	Proposed subdivision name if not within a 							previously platted subdivision.

3.	Ownership.

a.	Name and address, including telephone number 						of legal owner(s) of the property, or their 						agent.

b.	Citation and location of any existing legal 						rights-of-way or easements affecting the 							property.

c.	Existing covenants to which the property is 						subject, if any.

103 – 69

6 - 103 - 13       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 13

d.	Name and address, telephone number, and 							registration number, and seal of the 								professional engineer and/or surveyor 								responsible for subdivision design, for the 						design of public improvements, and for 							surveys.

4.	Descriptions.  Location of property by lot or 						section, township, range, and civil township (metes 					and bounds), with acreage.

5.	Graphic scale, north arrow and date shall be 						included.

6.	Features.  The primary plat shall show the 							following:

a.	Location of property lines, existing 								easements, burial grounds, railroad rights-of-						way, watercourses, and existing wooded areas, 						and names of all existing and/or platted 							streets or other public ways within the tract.

b.	The location of property with respect to 							surrounding property and streets, including

the uses of all adjoining property, the names 						of adjoining developments, and the names of 						adjoining	streets.

c.	Location, size, invert, elevations, and slopes 						of existing sewers, water mains, culverts, and 						other underground structures within the tract 						and existing permanent buildings and utility 						poles on the tract.

d.	Approximate topographic contours shown at five 						(5) foot intervals in rolling or hilly terrain 						and two (2) foot intervals in level terrain, 						referenced to sea-level datum and an 								established bench mark (may be waived on minor 						subdivisions).

e.	The intended location and width of proposed 						streets, and traffic control devices.

f.	Proposals for connection with existing water supply and sanitary sewer systems, or alternative means of providing water supply and sanitary waste disposal and treatment; preliminary provisions for collecting and discharging surface water drainage.

103-70

6 - 103 - 13       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 13

g.	The intended location, dimensions and areas of 						all proposed or existing lots.

h.	The intended location, dimensions, and areas 						of all parcels of land proposed to be set 							aside for park	or playground use or other 							public use, or for the use of property owners 						in the proposed subdivision.

i.	The location of temporary stakes to enable the 						local officials to find and appraise features 						of the proposed layout in the field, if 							needed.

j.	Whenever the primary plat covers only a part 						of an applicant's contiguous holdings, the 							applicant shall submit, at the scale of no 							more than one (1) inch equals two hundred 							(200) feet, a sketch of the entire holdings, 						including the proposed subdivision area, showing						an indication of the probable future 								street and drainage systems, for the remaining 						portion of the tract.

k.	All easements and any limitations of such 							easements.

l.	Building setback lines with dimensions.

m.	Lot areas in square feet.

n.	Typical features of driveway entrances.

o.	Zoning or land use regulations, which may apply, 						to the land.

p.	Ratios, if required by Zoning Ordinance:

-	Street right-of-way to gross area of 								tract;

-	Dwelling Units to gross area of tract;

-	Dwelling Units to net gross area of 								tract.

103 - 71

6 - 103 - 13       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 13

B.	Secondary Plat.

1.	General. A registered land surveyor shall prepare the secondary plat or engineer at a scale of not more than one (1) inch equals one hundred (100) feet.  The sheets shall be numbered in sequence if 	more than one (1) sheet is used.  All sheets shall be of such size as is acceptable for filing in the office of the County Recorder, but shall not be larger than twenty-seven inches by thirty-six 	inches (27" x 36").  The map prepared for the secondary plat shall be drawn on drafting paper or reproducible Mylar.

2.	Features.  The secondary plat shall show the 						following:

a.	The dimensions of all boundary lines of the 						property expressed in feet and hundredths of a 						foot, the bearings of all lines to a minimum 						of one-half (1/2) minute.  Location by 							Section, Quarter Section, Township, Range, 							Civil Township, County, and State.

b.	The location and width of all proposed rights-						of-ways, easements, alleys, and other public 						ways, and building setback lines.  Street 							names and/or numbers to be indicated.

c.	The locations, dimensions, and areas of all 						proposed to existing lots including dimensions 						of all lot lines expressed in feet and 							hundredths of a foot, and bearings of all 							lines to a minimum of one-half (1/2) minute.

d.	The location and dimensions of all property 						proposed to be set aside for park or play-

ground use, or other public or private 					reservation, with designation of the purpose 				thereof, and conditions, if any, of dedication 				or reservation.

e.	The name(s) and address (es) of the owner of 						land to be subdivided; the name and address of 						the subdivider, if other than the owner; and 						the name, seal, registration number and 							address of the land surveyor and/or engineer.  						Also, citation of last instrument conveying 						title to each parcel of property involved, 							giving grantor, date, and land record 								reference.

103 - 72

6 - 103 - 13       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 14

f.	Legal description of subdivision.

g.	The date, north arrow, and scale.

h.	Sufficient data acceptable to the Governmental 						Engineer to determine readily the location, 						bearing, and length of lines for reproduction 						of such lines upon the ground.

i.	The location of all proposed and existing 							monuments.

j.	Name of the subdivision.

k.	Lot areas in square feet.

l.	Protective covenants attached to plat.

m.	Notice of High Aircraft Noise Levels on 							Secondary Plat.  For subdivisions located 							within any noise overlay zone as established 						by the zoning ordinance or noise contour maps 						as adopted by the Hulman Regional Airport    						F.A.R. Part 150 Noise Compatibility Study 							Noise Exposure Maps a notice of potentially 						high aircraft noise levels shall be affixed to 						and recorded with secondary plat.  The notice 						shall be worded as follows:  "NOTE:  All or 						part of this subdivision is located in an area 						potentially subject to aircraft noise levels 						high enough to annoy users of the property and 						interfere with its unrestricted use.  Contact 						the Vigo County Area Planning Commission or 						the Hulman Regional Airport Authority for  							information regarding the most recently calculated					levels of current and forecast aircraft noise						levels on the property.

n.	The lack if information under any item 							specified herein, or improper information 							supplied by the applicant, shall be cause for 						disapproval of a secondary plat.

103 - 73

6 - 103 - 14       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 14

6 - 103 - 14   Soil and Land Use Data.  The following information 		     shall be presented with the Primary Plat.

A.	Subsurface.

1.	Soil Conditions.  The subdivider may submit 						percolation data as to location, and the rates of 					each test hole.  In addition, soil types having 						severe limitations or soil absorption sewage 						disposal systems, as designated by the National 						Cooperative Soil Survey, shall not be used for soil 					absorption sewage disposal facilities unless the 						Area Plan Commission, The Vigo County Soil and 						Water Conservation District representatives and the 					Vigo County Board of Health representative agree 						that such planned corrective measures have been 						taken so as to enable adequate and safe sewage 						disposal.  The Developer will be notified during 						the Pre-application Conference of those areas 						possessing severe limitations upon absorption 						sewage disposal systems.

2.	Subsurface Strata.  The location and results of 						tests made to ascertain subsurface soil, rock, and 					ground water conditions should be documented.

3.	Ground Water.  The depth to ground water shall be 					reported to the Commission unless test pits area 						dry at a depth of 5 feet.

B.	Surface Conditions.

1.	Natural Features.  Natural features such as watercourses, marshes, rock outcropping, lakes, wooded areas, isolated preserve able trees one foot or more in diameter shall be identified on a plan drawing.

2.	Man-made Features.  Man-made features such as 						houses, barns, shacks, electrical transmission 						lines, and other significant features shall be 						identified on a plan drawing.

C.	Conditions on Adjacent Land to be reported.

1.	Topography.

a.	Approximate direction and gradient of ground 						slope, including any embankments or retaining 						walls.

103 - 74

6 - 103 - 14       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 15

b.	Character and location of major buildings, 							railroads, power lines, towers, and other 							nearby non-residential land uses or adverse 						influences.

c.	Approximate area of off-site water shed 							draining into tract.

2.	Owners.

a.	Owners and present usage of adjacent land.

b.	Identification of adjacent platted land by 							subdivision plat name, recording date, and 							number.

3.	Zoning.  Zoning on and adjacent to the tract if 						applicable.

4.	Public Improvement.  Proposed public improvements, 					such as highways or other major improvements 						planned by public authorities for future 							construction on or near the tract.

D.	Sediment Control.  When development is proposed in areas 				designate by Vigo County Soil and Water Conservation 					District as being prone to sedimentation and erosion, 					the developer must submit with his preliminary plat, a 				statement that sediment and erosion control methods 					shall be provided prior to any clearing, grading, or 					construction.  The developer after consultation with 					Vigo County Soil and Water Conservation District rep-					representatives and Area Planning representatives will 				submit two copies of his proposed plan for sediment and 				erosion control.  The Area Planning Department shall 					request a review by the Soil and Water Conservation 					District for their recommendations before approval of 					the primary plat.

E.	Landscaping.

1.	Landscaping is encouraged in order to enhance the 					aesthetics of the subdivision.

2.	The subdivider shall submit his proposal and 						schedule for landscaping to the commission prior to 					filing the secondary plat.

103 - 75

6 - 103 - 14       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 15

3.	Use of protective screening is necessary in 						securing a reasonably effective physical barrier 						between different types of land uses.  Appropriate 					use of fences, wall or plant materials, or in 						combinations may be required by the Area Plan 						Commission.

4.	No cut trees, timber, debris, earth, rocks, stones, 					soil, junk, rubbish, or other materials of any kind 					shall be buried in any land, or left deposited on 					any lot or street at the time the buildings are 						ready for occupancy.

6 - 103 - 15  Fees.

A.	This subdivision ordinance establishes a uniform 						schedule of fees proportionate to the cost of checking 				and verifying the proposed plats.  An applicant shall 					pay the specified fee in full at the time of filing his 				application by check or money order in accordance with 				the Schedule of Fees below.  The check or money order 					shall be made payable to the Area Plan Commission of 					Vigo County.

------------------------------------------------------------------

Amendment for the unincorporated areas of Vigo County

Amendment for the City of Terre Haute

A.	This subdivision ordinance establishes a uniform 						schedule of fees proportionate to the cost of checking 				and verifying the proposed plats.  An applicant shall 					pay the specified fee in full at the time of filing his 				application by check or money order in accordance with 				the Schedule of Fees below.  The check or money order 					shall be made payable to Vigo County.

------------------------------------------------------------------

103 - 76

6 - 103 - 15       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 16

TABLE 8

SCHEDULE OF FEES

------------------------------------------------------------------

PREAPPLICATION CONFERENCE.  NO CHARGE

------------------------------------------------------------------

PRIMARY AND SECONDARY APPLICATION SCHEDULE OF FEES:

------------------------------------------------------------------

NO.	Type of Intensity of	 	Primary	          Secondary

Subdividing			Application $        Application $

------------------------------------------------------------------

1	New Subdivision			 $50 and $1 per     $25 per plat

(See No. 5)			 lot or dwelling

unit whichever

is greater

------------------------------------------------------------------

2	Resubdivision			 $50 and $1 per     $25 per plat

(See No. 5)			 lot or dwelling

unit whichever

is greater

------------------------------------------------------------------

3	Partial Mapping		 	$50 and $1 per     $25 per plat

lot mapped

------------------------------------------------------------------

4	Legal Advertising		 $10

(Major Only)

------------------------------------------------------------------

5	Private Driveways Access-	$5 per inspection 	First inspection

Subdivision Street 		(one driveway) 		is free, $5 for each

Where New Development		and $1 for each   	call back per driveway

Occurs (Fees Apply to	 	additional driveway						No. 1, 2, & 3 in this	 	in the same	subdivision

Schedule)

------------------------------------------------------------------

6 - 103 - 16  Release of Bonding.

A.	Assurance for completion.

1.	As provided in Chapter 6-103-9(A-16) the subdivider 					has the option of either installing all require 						public improvements prior to secondary approval of 					the plat, or in lieu of installation, posting a 						bond in an amount sufficient to assure completion 					of said improvements.

103 - 77

6 - 103 - 16       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 16

2.	Posting of Performance Bond.

a.	The Area Plan Commission, at its discretion, 						may waive the requirement that the applicant

complete and dedicate all public improvements 						prior to the signing of the subdivision plat, 						and that, as an alternative, the applicant 							post a performance bond.

3.	Completion of Improvement.

a.	The applicant shall build and pay all costs 						for temporary improvements required by the 							Area Plan Commission and shall maintain same 						for the period specified by the Area Plan 							Commission.  Prior to construction of any 							temporary facility or improvement, the 							developer shall file with the local government 						a separate suitable bond for temporary 							facilities, which bond shall ensure that the 						temporary facilities will be properly 								constructed, maintained, and removed.

b.	For subdivision for which no performance bond 						has been posted, if the improvements are not 						completed within the period specified by the 						Area Plan Commission in the resolution approving						the plat, the approval shall be deemed to							have expired, and the secondary plat void.

4.	Acceptance of Dedication Offers.

a.	Acceptance of formal offers of dedication of 						streets, public areas, easements, and parks 						shall be by official action of the Board of 						Public Works, Town Board or Board of County 						Commissioners.

b.	The approval by the Plan Commission of a 							subdivision plat shall not be deemed to 							constitute or imply the acceptance by the 							local government of any street easements, or 						other public areas shown on said plat.

5.	Release or Reduction of Performance Bond.

a.	The Governing Body shall not accept dedication 						of neither required improvements, nor release nor 					reduce a performance bond, until the govern-						mental engineer has submitted a certificate							stating that all required improvements have 						been satisfactorily completed.

103 - 78

6 - 103 - 16       SUBDIVISION CONTROL ORDINANCE      6 - 103 - 19

b.	A performance bond may be reduced upon actual dedication and acceptance of public improvements 	and then only to the ratio that the dedicated public improvements bear to the total public improvements for the plat.  In no event shall a performance bond be reduced below ten (10) percent of the principal amount until all improvements have been completed and accepted.

B.	Deferral or Waiver of Improvements.

1.	General Conditions.  The Area Plan Commission may 					defer or waive at the time of secondary approval, 					subject to appropriate conditions, the provisions 					of any or all such improvements as, in its judgment,					are not requisite in the interest of public 						health, safety, and general welfare, or which are 					inappropriate because of inadequacy or lack of 						connecting facilities.

6 - 103 - 17  Repealer.

A.  	All ordinance or parts of ordinances in conflict with 					the provisions of this ordinance are hereby repealed.

6 - 103 - 18  Severabilty Clause.

A.	If any section, provision, or part of this ordinance is 				adjudged invalid or unconstitutional, such adjudication 				shall not affect the validity of the ordinance as a 					whole or any Section, provision, or part thereof not 					adjudged invalid or unconstitutional.

6 - 103 - 19  Effective Date.

A.	Unincorporated Areas of Vigo County, Indiana and the 					City of Terre Haute, Indiana, December 28, 1988.  The 					Towns of Riley, Seelyville, Indiana, February 15, 1989.

103 - 79